SEWAGE EFFLUENT STANDARDS

(Extracted from the Jamaican National Sewage Effluent Standards, 1996)

# Immediate Technology Based Effluent Standards – Existing Plants

| ## PARAMETER | ### EFFLUENT LIMIT |
| --- | --- |
| BOD5 TSS Nitrates (as Nitrogen) Phosphates COD PH Faecal Coliform Residual Chlorine | 20 mg/l 30 mg/l 30 mg/l 10 mg/l 100 mg/l 6-9 1000 MPN/100ml 1.5 mg/l |

Proposed Sewage Effluent Standards – New Plants 1

| #### PARAMETER | #### EFFLUENT LIMIT |
| --- | --- |
| BOD TSS Total Nitrogen Phosphates COD PH Faecal Coliform Residual Chlorine | 20 mg/l 20 mg/l 10 mg/l 4 mg/l 100 mg/l 6-9 1000 MPN/100ml 1.5 mg/l |

1 A plant is considered new if it was constructed and commissioned on or after January 1, 1997 (i.e. the commencement of the NRCA’s Permit and Licence System)

PARAMETERS IN STANDARDS

The parameters for which the standard establishes limits are:

BOD                                    	Biological Oxygen Demand

TSS                                     	Total Suspended Solids

Nitrates                               	Nitrogen in Nitrate and Nitrite

Phosphate                           	Phosphours in Phosphate

COD                                   	Chemical Oxygen Demand

pH                                       	Acidity or Alkanity

Faecal Coliform		Coliform Bacteria

MPN  			Most Probable Number

Residual Chlorine*

*Where natural treatment systems are designed to reduce coliform levels without the use of chlorine then the residual chlorine criteria would not apply