TRADE EFFLUENT STANDARDS

(Extracted from the Jamaican National Trade Effluent Standards 1995)

PARAMETER                                                STANDARD LIMIT

### Ammonia /Ammonium                                      1.0 mg/l

Barium                                                               5.0 mg/l

Beryllium                                                           0.5 mg/l

Boron                                                                 5.0 mg/l

### Calcium                                                              No Standard

Chloride                                                              300 mg/l

Colour                                                                 100 TCU

### Detergent                                                            15 mg/l or

<0.015 kg/ 1000 kg product

Fluoride                                                               3.0 mg/l

Iron                                                                      3.0 mg/l

Magnesium                                                          No standard

Manganese                                                           1.0 mg/l

Nitrate (as Nitrate and Nitrite)                             10 mg/l

Oil and grease                                                      10 mg/l or

<0.01kg/ 1000 kg product

pH                                                                        6.5 – 8.5

Phenols                                                                0.1 mg/l

Phosphate					     5.0 mg/l

Sodium					      100 mg/l

Sulphate                                                                250 mg/l

Sulphide                                                                0.2 mg/l

TDS                                                                      1000 mg/l

Temperature                                                          2 C +/- average ambient temperature

TOC                                                                      100 mg/l

TSS                                                                       All times <150 mg/l

Monthly average 50 mg/l

##### Heavy Metals

### Arsenic                                                                  0.5 mg/l

Cadmium                                                              0.1 mg/l

Chromium                                                             1.0 mg/l

Copper                                                                  0.1 mg/l

Cyanide (Free HCN)				      0.1 mg/l

(Total CN)				      0.2 mg/l

Lead      					      0.1 mg/l

Mercury 					      0.02 mg/l

Nickel						      1.0 mg/l

Selenium					     0.5 mg/l

Silver						      0.1 mg/l

Tin						      No standard

Zinc                                                                      1.5 mg/l

Total Heavy Metals                                              2.0 mg/l

##### STREAM LOADING

BOD5                                                                       < 30 mg/l

COD                                                                         < 0.1kg/ 1000kg product

or < 100 mg/l

DO                                                                             > 4 mg/l

###### BACTERIOLOGY

## Total Coliform                                                            < 500 MPN/ 100ml

### Feacal  Coliform                                                       	<100 MPN/ 100ml

ACRONYMS

FOR   PARAMETRS   USED   IN   STANDARDS

BOD	Biochemical Oxygen Demand

COD	Chemical Oxygen Demand

DO	Dissolve Oxygen

MPN	Most Probable Number

TCU	Total Colour Unit

TDS	Total Dissolved Solids

TOC	Total Organic Carbon

TSS	Total Suspended Solids

FURTHER   CLARIFICATION

### Nitrates			Refers to nitrogen in nitrates and nitrites

Phosphates		Refers to phosphorous in phosphates

pH			Measures acidity or alkalinity

Residual Chlorine	Where natural treatment systems are designed to reduce coliform levels without

the use of chlorine then the residual chlorine criteria would not apply