Health and Safety Program

Jamaica Project

TABLE OF CONTENTS

1	General	4

2	Safety	4

2.1	Organization	4

2.2	safety manual	5

2.3	Monitoring	5

2.4	Reporting	5

2.5	Enforcement of the Safety Management Plan, the safety manual and the safety rules and practices	Error! Bookmark not defined.

2.6	Compliance with Applicable Laws	5

3	Safety Rules and Practices	6

3.1.1	Hazards assessment and method statement	6

3.1.2	Permits/procedures	8

3.1.3	Safe Behavior	8

3.1.4	Housekeeping	8

3.1.5	Personal Protective Equipment (PPE)	9

3.1.6	Lock out/tag out Procedure (LOTO)	9

3.1.7	Working at height	10

3.1.7.1	Working on roof	10

3.1.7.2	Working on platforms:	11

3.1.7.3	Working on structural steel.	12

3.1.7.4	Scaffolds:	13

3.1.8	Excavations	14

3.1.9	Welding and cutting	15

3.1.10	Crane and mobile equipment	16

3.1.11	Temporary power supply	173.1.12	Blasting	18

3.1.13	Hazardous materials	18

3.1.14	Evacuation plan and procedures	18

3.1.15	Fire prevention	19

3.1.16	First aid facilities	20

3.1.17	Safety training	20

3.1.18	Other General Safety Aspects	21

4	Site Practices and Conditions	22

4.1	Generalities	22

4.2	Fencing	22

4.3	Site access and circulation	22

4.4	Speed limit and roads protection	23

4.5	Site lighting	23

4.6	Site housekeeping	23

4.7	Contractor temporary facilities

4.8	Utilities connecting points

4.9	Site guarding	24

4.10	Coordination with other contractors	24

4.11	Preservation of Employer property	24

4.12	Ammunitions, drugs, weapons, explosives	24

4.13	Working hours	25

4.14	Fossils and antiquities	25

4.15	Noise level	25

- General health and safety plan

- General

All requirements within this safety and health DCS (Design Criteria and Standards) are the minimum requirements. If any local standard requires a higher level the contractor is obliged to fulfill. Besides that, the Chinese safety and health standards and requirements have to be respected.

Each individual person of SINOMA ‘s who work for this project must enforce about these safety regulations.

# Safety

## Organization

SINOMA Q&S D

Project Manager

Project Safety Manager

Civil construction team--part-time safety personnel

Equipment

Installation

team—part-time safety personnel

Electrical installation team—part –time safety personnel

Sub-contractor team—full-time safety personnel

Project manager: Project manager is responsible for the entire process of the project and all safety management within the scope of the project.

Safety manager: Safety manager is responsible for site safety during construction activities.

Full-time safety person: Guided by the safety manager, full-time personnel are responsible for the daily routine of safe production.

Department manager: Department manager is responsible for the safety of his/her department in an all-round way.

Departmental full-time and part-time safety person: Those persons are responsible for departmental daily routine of safety work.

Subcontractor full-time safety manager: He/she is responsible for the safety work within subcontractor’s scope of works.

Subcontractor full-time safety personnel: They are responsible for the daily routine of safety work within subcontractor’s scope of work.

The Contractor Site Manager will be responsible for organizing, managing, coordinating and controlling the Contractor’s safety activities.

This Safety Manager, or his designated deputy, have the credentials(in china ) and/or experience.

## Safety manual

## Monitoring

Each weekly site meeting will start with Safety on the agenda.

Specific safety meetings will be held when necessary at the request of the Contractor or of the Employer’s Site Manager.

The Employer’s Safety Officer shall carry inspections of the works. When the Contractor or the Contractor’s personnel is invited to these inspections, it is mandatory for them to attend.

The Employer reserves the right to audit the safety of the site. This encompasses actual construction operations, offices, shops, warehouses and any other facility as well as administrative documents.

The Contractor shall comply with this request.

The Employer’s Safety Officer is entitled to request any documents required to certify the conformity of the equipment, tools and materials used by the Contractor. The Contractor shall comply with this request.

## Reporting

## Each accident or near miss accident must be reported.

This includes bodily injury as well as damages to equipment or property of Employer or third party.

Any other accident must be reported the day it happened to the Employer’s Site Manager.

a written report issued to the Employer’s Site Manager within one working day of the happening of the accident.

For fatal and Lost Time Accident (LTA), the Contractor must perform a root cause analysis to understand the real causes of the accident and to take the appropriate measures. The Employer’s Safety Officer will train, if required, the Contractor for such analysis.

Near miss accident which could have lead to a fatal accident must be reported according to the fatal accident rule.

All the Contractor findings must be available to the Employer’ Safety Officer.

The project safety manager will provide the Employer at the end of every month, within 2 working days, together with the monthly summary of all LTA, near misses, and fatal accidents.

## Compliance with Applicable Laws  适用法律的遵守

full compliance with all applicable laws and regulations and directives of governmental agencies having jurisdiction over the site.

# Safety Rules and Practices

# These rules apply from the inception of the works till the Provisional Acceptance.

During commissioning these rules may be completed or superseded by the plant “Safety Operational Procedures”.

### Hazards assessment and method statement  危害评价和方法描述

The first step toward safety is a proper planning of the tasks to be done and an identification of the potential hazards of these tasks.

Therefore the Contractor shall conduct a risk assessment for each major or repetitive construction activity, including commissioning.

The Contractor will select the tasks for which he plans to prepare a risk assessment and submit the list to the Employer’s Site Manager Comments.

The risk assessment findings will be summarized in a risk assessment form which should include a description of the operation, a description of the hazards of the operation, the anticipated effects, the numbers of employees exposed to the hazard, the injury probability, the risk rating and the measures taken to eliminate or alleviate the risk.

The risks assessments will form the basis for method statements.

Once the risk assessment is completed for a task, we shall implement a risk control/elimination approach.

The levers on which we can act are:

- Elimination of the risk

- Control of the risk by acting on:

Site engineering/equipment

Procedures

Personal Protective Equipment

Behavior

we will produce construction project (method statements) for all major construction activities.

Furthermore these method statements are mandatory for any works that have a risk rating from substantial to high and for any scaffold work at height greater than 2 meters.

Based on the above, the Contractor will select the tasks for which he plans to prepare a method statement and will submit the list to the Employer’s Site Manager’s approval.

(Refer to guidelines to prepare a method statement given below)

Once agreed upon the method statement must be strictly implemented and followed by the Contractor, any deviation will have to be discussed with Employer beforehand.

Method statements guideline

A method statement must describe clearly and thoroughly how the Contractor intends to perform a specific task. It should give an answer to the following simple questions: Where, How, When, With what, By whom. More specifically the method statement shall cover the following points: - title - description - explanation of process - location - time/duration - sequence - resources - personnel - supervision - plant/equipment - materials - risk assessment - complete risk assessment forms for all tasks - access/egress - risk for public - noise, OSHA etc - control measures to be used - permits - special training - special safety - PPE (Personal Protective Equipment ) requirements - emergency arrangements - Rescue - Special requirements The method statement shall be reviewed and approved by all concerned parties within the Contractor’s organization and signed by the Contractor’s Safety Manager then submitted to the Employer’s Site Manager for comments. The Contractor is then responsible to check that actual operations follow the method statement and to inform the Employer in cases of changes.

### Permits/procedures

Depending on the type of the works and on the risk level, a system of permits/procedures may be introduced.

At a minimum the following permits/procedures shall be required:

- to enter and to work in confined spaces (cyclones, tanks, bins, silos)

- for works involving chemical and hazardous materials

- to work with open flames, for welding and to work in hazardous environment when required and agreed upon by both parties.

- to work on installation under pressure

- for disassembly or demolishing

- to work in deep excavation

- etc.

We will issue the permits procedure. A copy of the permit/procedure must always be posted or kept in order to allow the Employer to check it.

If and when the works are interfacing with the plant normal activities these permits procedure require the approval and the signature of the Employer’s Site Manager.

### Safe Behavior

Rough and tumble, fighting and disruptive behavior are prohibited on Employer’s property or within the site fence.

Caution must be used when the ground surface is slick or uneven.

Jumping on or off equipment, stockpiles or other elevated surfaces is forbidden

Obey non smoking signs

Suitable clothing must be worn, without loose parts

Jewelry shall not be worn when operating rotary equipment

### Housekeeping

### Housekeeping in of paramount importance for everyday site safety

。

In order to prevent accidents We will permanently provide a safe working environment and shall coordinate with any other contractor on site.

Housekeeping will be strictly enforced.

- Pedestrian walkways shall be clearly identified, marked and protected from the traffic of construction equipment.

- Work areas shall be maintained in a clean, neat and orderly manner to minimize potential trip/fall hazards.

- Standing water, frost or ice must be cleaned.

- Walkways, working platforms and working areas shall be clean and free from any impediment (water, dirt, ice, piping, cables, construction materials, construction materiel, etc.) in order to maintain safe working conditions.

- Worn out, defective or non functioning equipment or parts thereof shall be removed from the site and properly discarded.

- Fire extinguishers, fire hydrants, fire alarms, electrical panels or circuit breakers accesses must always be kept free.

- Any spillage of hazardous material must be immediately cleaned.

- The site must be cleaned on a daily basis, wastes and garbage must be stored in appropriate space before being disposed of at licensed dump sites.

### Personal Protective Equipment (PPE)

We are  responsible for ensuring that the PPE safety equipment is in conformance with all pertinent laws, norms and standards.

It is mandatory to wear PPE in accordance with the following rules, except within canteen and/or protected break areas, restrooms and other office areas. No other exceptions will be authorized. Violation will lead to disciplinary actions.

Approved safety hats and safety shoes, fit for the job undertaken, must be worn at all time and everywhere.

- Eye protection of approved and required type (goggles or shield) must be worn when grinding, chipping, welding, working with chemical substances or engaged in any activity hazardous to eyes (dusty environment for example).

- Hearing protection is required when employees are subject to noise levels above the exposure limits defined by local laws.

- Respiratory protection must be worn when engaged in works in a dusty or gaseous environment.

- Gloves must be worn when necessary to protect the hands.

- Specific heat resisting equipment must be worn when there is a risk of contact with hot material.

- Individual, personal padlocks must be used for LOTO (lock out – tag out procedure).

- High visibility waistcoat must be worn by signal men and everyone else when working in low lighting conditions.

- Fall protection harnesses must be worn when working above 2 meter in areas without proper collective fall protection (example handrails or safety nets), adequate lifelines must be installed and the safety harnesses must be clipped to the lifelines. This is mandatory as well when building or dismantling scaffoldings.

- Harnesses must be designed for 100% tie-off, i.e. with two lanyards.

- Double independent “lines” are required when working on the Kiln and Rotary Mills: one “life line” for workers safety, and another independent “tools line” for securing tools, so workers do not have to untie from the safety line when walking on the shell and passing over tools resting on the shell and secured to the tools line.

- Safety belts are not an approved substitute.

### Lock out/tag out Procedure (LOTO) 隔离程序(LOTO)

The LOTO procedure is mandatory when working on live equipment which could cause injury by contact with energized parts, release of fluid under pressure or moving mechanical parts.

The purpose of this procedure is to establish the general rules of operation, switching of electrical equipment or equipment under pressure or carrying dangerous fluids or gas and securing it against unintended or autonomous re-start. This procedure shall determine the basic principles which the Contractor has to follow.

We will  prepare the detailed procedure according to prevalent and Chinese rules and regulations and present it to the Employer for approval.

This procedure will be implemented permanently starting with no load tests

In general,

- Before starting works on electric drives these shall be switched off and secured against re-start.

- The permit for disconnecting the Disconnecting Switch (DS) is granted by the CCR operator or the shift leader. The permit requires written or electronic form.

- Disconnecting without operator’s permit is forbidden.

- Disconnecting and securing electric drives with a Disconnecting Switch (DS) shall be performed by all authorized Plant and Contractor’s employees only.

Each employee must be trained in the area of drives switching off and blocking. The training must be confirmed with signature of the trained person.

The person blocking a device with Disconnecting Switch (DS) is responsible for switching the device off and protecting it against re-start.

Any request for specific and limited exceptions during commissioning must be discussed and agreed between Employer’s Safety Officer and Contractor’s Safety Manager.

### Working at height  高处作业

This encompasses working on roof, working on platforms, working on structural steel and working on scaffolds.

#### Working on roof   屋顶作业

A risk assessment shall be carried for all roof work.

The minimum safety conditions as given in the below table “Safety on Roofs” shall be applied.

Safety On Roofs

All roof work is dangerous and a risk assessment and a method statement are mandatory. Good planning can reduce the risks and facilitate the work. The main following points shall be addressed: - Accesses Safe and proper access means shall be foreseen. Traveling about the roof should be kept to a minimum. - Materials handling Manual handling should be reduced as far as possible. - Edge protection In case of risk of falling more than 2 meter there shall be an edge protection meeting the appropriate standards and at least equivalent to: A handrail at 1 meter above the edge resisting to horizontal shock as defined in the local laws. An intermediate guardrail (maximum gap between guardrail, handrail and toe-board approximate 0.5 meter) A toe-board - Fall arrest equipment If no other ways is possible or practicable then the Contractor shall supply and install safety nets or safety harnesses and lifelines. - Falling materials Nothing shall ever been thrown from a roof. The areas underneath or adjacent to the Works shall be fenced. If this is not possible the Contractor shall consider how to protect the personnel by using debris nets covered walkways or similar safeguards. - Weather conditions If practicable there should be no work on roof in adverse weather conditions (ice, rain, wind, excessive heat) or adequate protection shall be used. - Fragile roofs A fragile roof is a roof, which will no support the weight of a person. In case of doubt the roof shall be treated as fragile until otherwise demonstrated. Besides all precautions describe above a permit system shall be implemented and measures to provide a safe working area shall be implemented (temporary strengthening).

#### Working on platforms:   平台作业

The platforms could be independent of be part of a scaffold. In both cases the rules pertaining to scaffolds shall be followed (handrails, toe-boards, access, as described in the below given table “Safety on Platforms and Elevated Floors” shall be applied.

Safety on Platforms and Elevated Floors

The following apply to temporary working platforms; future permanent platforms used as a working surface during construction and elevated floors used as temporary platforms during construction. Depending on the circumstances, working on a platform can be dangerous. In this case a risk assessment and a method statement are mandatory. Good planning can reduce the risks and facilitate the work. The main following points shall be addressed: - Accesses Safe and proper access means shall be foreseen. Traveling about the platform should be kept to a minimum. - Materials handling Manual handling should be reduced as far as possible. - Edge protection In case of risk of falling more than 2 meter there shall be an edge protection meeting the appropriate standards and at least equivalent to: A handrail at 1 meter above the edge resisting to horizontal shock as defined in local regulations. An intermediate guardrail (maximum gap between guardrail, handrail and toe-board less than 0.5 meter) A toe-board - Fall arrest equipment If no other ways is possible or practicable then the Contractor shall supply and install safety nets or safety harnesses and lifelines. - Falling materials Nothing shall ever been thrown from a platform or a floor. The areas underneath or adjacent to the works shall be fenced. If this is not possible the Contractor shall consider how to protect the personnel by using debris nets covered walkways or similar safeguards. The attention of the Contractor is drawn particularly to the areas around the preheater tower where adequate protection from falling materials have to be foreseen (covered walkways, debris nets, temporary canopies at different levels, etc.) - Weather conditions If practicable there should be no work on roof in adverse weather conditions (ice, rain, wind, excessive heat) or adequate protection shall be used.

#### Working on structural steel.   The safety conditions as described in the below given table “Safety on Structural Steel Erection” shall be implemented by the Contractor as minimum rules to abide by.

Safety on Structural Steel Erection

Risk assessments and method statements shall be prepared for all steel structure erection works. Any deviation to this rule has to be agreed by the Employer. Good planning can reduce the risks and facilitate the work and the Contractor must plan in detail his operations before starting the works. The following points must be taken into consideration by the Contractor: - PPE Suitable PPE shall be worn, safety shoes must be adapted to the task, and helmets must have a chin-strap. - Access Access to the working place shall be by fixed safe ladder, scaffolds, “cherry-picker truck”, or crane basket. - Beam walking Is strictly forbidden. Either a safety net must be installed beforehand or a lifeline. In this case safety harnesses must be worn. All other applicable stipulations from the chapters Safety on roof and Safety on platforms shall be adhered to.

#### Scaffolds:  All fix or mobile scaffolding must comply with local regulations or at least as described in the below given table “Scaffolding Safety Procedures”. Scaffolding must be properly designed and inspected before being put in use. When in use, weekly inspection is required.

No access is allowed if the inspection tag described in the table is missing.

“System scaffolding” or steel/aluminum pipes scaffolding shall be preferred.

Any temporary gap on roof, platform or floor shall be either covered or protected by adequate handrails and toe-board.

Scaffolding Safety Procedures

The laws and regulations in china for the scaffoldings will be applied and prevail the following rules for scaffoldings. The following rules are the minimum expected to be followed on site. All relevant local rules must be adhered to. “System scaffoldings” are preferred and shall be used wherever possible. The scaffold must be fit to the task being undertaken. Harnesses: - All those involved in scaffolding operations shall wear and use fall arrest equipment, and shall have received appropriate and recorded training in the use, inspection and maintenance of such equipment. - When clipping on, a suitable anchor point shall be used. - When working at or above 2 meter, workers must clip on if: - They are not working within a protected area (60 cm wide platform and a single handrail on each non protected side) - They are climbing up and down the scaffold structure - They are working off the scaffold structure - They are raising or lowering materials or boards within a safe handling platform (double handrail provided on each unprotected side, toe-board). Ladder access: - Ladders shall be included as early as possible in the erection process and removed as late as possible in the dismantling process to reduce the risk of climbing the structure. - Incorporation of protected ladder bays is mandatory Decking - The minimum width of the working platform shall be 60 cm, with gaps smaller than 3 cm between boards. - Adequate boards shall be used in order to allow sufficient space on the edges to allow access to the ledge tube at platform level with a harness hook when there is no other way to clip the harness. This space shall be less than 20 cm to prevent falling. - Platform boards shall be design to support the intended loads, they shall be fixed in a stable fashion.

### Excavations  All excavation must be adequately protected, according to the below given table “Safety in Excavations”.

Safety in Excavations

A competent person must supervise the construction, alteration and backfilling of excavation and the need for support. All excavations shall be fenced. Care will be taken to keep traffic away from excavations .When excavations are close to traffic areas specific measures will have to be taken by Contractor. Therefore before starting any excavation the Contractor shall plan against: - collapse of the sides - materials falling inside the excavation - people and vehicles falling - undermining adjacent structures - hitting underground services The Contractor shall make sure that all necessary supporting, draining and access equipment is on site before starting the work. Suitable escape means must be installed. Nobody must be left to work alone in an excavation deeper than 2 meter. A risk assessment and method statement shall be carried for every excavation deeper than 2 meter.

### Welding and cutting  Personnel performing welding and cutting shall be qualified and trained regarding the hazards associated with compressed gas cylinders and the appropriate emergency response procedures.

In specific cases a permit may be required (work in hazardous areas).

Adequate PPE shall be worn, goggles, shield, shoes, etc.

Screens shall be placed around the welding/cutting areas according to local rules.

The bottles shall be used and stored according to the below given table “Precautions: Using Compressed Gas Bottles and; Welding and Cutting”.

All other welding and cutting operations shall be done in accordance with the below given table “Precautions: Using Compressed Gas Bottles and; Welding and Cutting”.

Precautions: Using Compressed Gas Bottles and; Welding and Cutting

Compressed gas bottles: Must be fitted with flashback arrestor in gas hose close to a suitable pressure regulator and a non return valve shall be installed close to the entry point of the blowtorch. Must be stored at more than 10 meter of an open flame Must be stored away from any heat source warmer than 50°C Must be stored vertically or in an approved basket/carrier when in use Must not be stored near an electrical source nor where they could come in contact with electrical wiring. Empty bottles shall be stored separately from full bottles with their valves closed and shall be segregated by bottles content. Must conform to relevant local regulations Arc welding: Arc welder shall be adequately insulated or shall be protected not as to create electric shock to personnel. They shall be grounded. Rods shall be removed from the holder when personnel leave the work area. Welding and cutting: Whenever possible keep welding hoses or cables on overhead bracket to prevent tangling of the hoses or tripping on them. Protect hoses and cables from welding sparks and molten metal. Protect any flammable equipment, supplies, wastes or materials from sparks or molten metal. Suitable portable fire extinguishers shall be supplied at each welding working areas. A fire watch shall be implemented to check the site 30 minutes and 2 hours after welding is complete.

### Crane and mobile equipment  All equipment shall be in good state, properly maintained and have the proper required certificates

All equipment drivers shall have the adequate training and certificates.

If equipment is normally supplied with safety belts they shall be used at all time.

If the local regulations require back up horns, these shall be in proper working conditions and regularly inspected.

Riding on construction equipment by personnel is not permitted.

All equipment operators must follow the site speed regulations.

Trained signal men must be present permanently when cranes are working. If necessary, proper communication system must be provided by the Contractor.

The area around the crane and under the path of a crane load must be roped off with white and red streamers.

Crane outriggers must rest on adequately ground; the load must be evenly distributed by appropriate means.

Crane cables and slings must conform to local and Chinese regulations as well as the minimum stipulations of the below given table “Crane and Mobile Equipment”.

If a crane is used to carry workers in a basket the minimum stipulations of the below given table “Crane and Mobile Equipment” must be obeyed.

Crane and Mobile Equipment

Basket for personnel All baskets hung to a crane and used to carry personnel will be built with: - non slippery floor - toe-board - handrails - a metallic frame with 4 structural elements connected to a hanging ring having a safety factor as required by local regulations, or superior to 10, if there are no local rules. The number of personnel will be limited to 3 if nothing else is foreseen by local regulations. All personnel must wear a harness clipped on the basket structure. Slings and cables inspection All slings and cables must have been certified by a competent authority and must have a tag showing the maximum load allowed. All slings and cable must be visually inspected before each use. Any sling or cable badly damaged, cut, worn out or with broken strands will be rejected. Slings or cables must be used in a way to ensure the handling of the load without risk of slippage or fall. A signal man must be at hand when moving loads with slings or cables and the whole area must be roped off. All loads must be lifted vertically. If skewed lift cannot be avoided there shall be a Contractor’s supervisor managing it and taking the required special precautions.

The Contractor shall use only tested equipment, tools, chains, ropes, etc, and shall periodically test them to ensure good working condition of such equipment, tools, chains, ropes, etc.

Use of cranes and hoists including their attachments, anchorage and supports shall conform to the following standard conditions:

Cranes and hoists shall be of good mechanical construction, sound materials and adequate strength and shall be kept in good working orders.

Every rope used in hoisting or lowering materials or as means of suspension shall be of durable quality and adequate strength and free from defects. Frayed ropes shall not be used.

Every crane driver or hoist operator shall be properly trained and shall be conversant with safety regulations for the use of such equipment and rigging.

All cranes and hoists shall be clearly marked with the safe working load.

Motors, gearing, transmissions, electric wiring and other dangerous parts of hoists shall be provided with efficient safeguards. Rigging shall be by such means as to prevent the accidental falling of the suspended load.

### Temporary power supply

### The design, construction, operation and maintenance of temporary power supply must comply with all relevant regulations.

The Contractor must ensure that electrical boxes have dry wooden platforms, rubber mats or other non-conductive material under them to protect employees.

The Contractor shall ensure that all electrical boxes have a disconnection device for each branch circuit which must be labeled to show what each box and branch controls. These boxes must be permanently properly locked.

### Blasting

The Contractor shall be fully responsible for damages resulting from the use of explosives (i.e.: flying rocks, vibrations affecting works or existing structures, etc). The Contractor shall propose for every stage of the work:

- Program

- Location and orientation plan of the work face

- Drilling loading and blasting patterns.

- Method for scaling and grading of the slopes and walls.

- Safety procedures.

### Hazardous materials

### The Contractor shall provide to the Employer the Material Safety Data Sheets (MSDS) or equivalent for all hazardous substances that he supplies.

The MSDS shall bear the appropriate chemical abstract service number and the relevant classification and shall contain health, safety and other information concerning hazards and handling procedures consistent with environmental health standards prevailing at the site location or required by the Employer.

The Contractor shall store and dispose of hazardous or combustible materials according to applicable regulations. Storage areas shall be clearly marked. The Contractor shall provide the required approved fire fighting systems.

The Contractor shall take the necessary storage and disposal precautions to prevent any spillage from hazardous materials and more specifically from fuels, grease and oil. No drainage is allowed in the sewer system.

The Contractor shall clean and dispose of any spillage in accordance with local regulations. The spillage areas shall be made safe and clean.

When a spillage occurs in the Contractor’s areas, the Contractor shall immediately notify the Employer’s site manager and take all necessary actions to contain and dispose of such spillage at his own expense.

### Evacuation plan and procedures

### At the taking over of the site, the Contractor shall prepare the procedures and evacuation plan for the complete construction area, i.e. the Works, the Contractor’s offices, the Contractor’s warehouse and all areas under Contractor responsibility. This plan shall take into account any interference with the Employer’s operations. This plan shall be prepared and updated with the participation of the Employer’s Site Manager and Safety Officer.

Evacuation of the top levels of the preheater tower shall be considered.

When the plan is completed the Contractor will present it to the Employer. The plan will become effective after agreement by the Employer.

The safety and evacuation plan shall include and indicate:

- water hydrants

- main power switches

- entry and exit gates

- emergency accesses

- evacuation routes

- helicopter landing pad (if applicable)

- first aid facilities

- assembly point for site personnel

- etc.

We will maintain continuously free all escape routes defined on the evacuation plan. Escape routes shall be marked by distinct and adequate signage.

We will post this plan in its offices and in its subcontractor’s offices and shall communicate it to the Employer and to the fire brigade.

The plan shall be updated, if necessary, by us in the same conditions as for the initial preparation.

### Fire prevention  We will familiarize himself to the types of alarm and the sounds of such alarms at Site. We will also familiarize itself to the Site Safety and Evacuation plan, all relevant assembly points, fire exits, breathing apparatus, extinguishers and fire fighting equipment in, on or around the Site for use in emergencies. The Contractor must be provided with the necessary number of fire extinguishers. Arrangements are necessary with the local fire brigade in order to ensure an adequate fire protection for the site.

Fire blankets shall be kept in convenient locations.

Fire drills shall be conducted by us at regular intervals.

All work areas must be provided with the necessary number The Contractor as part of his scope of work accepts responsibility for ensuring that he and his subcontractors and suppliers comply in every respect with the contractual and governmental regulations concerning fire protection. He is furthermore responsible for ensuring that he co-ordinates his work in respect of fire protection with the other Contractors on Site.

Fire fighting equipment shall be serviced by the Contractor at the minimum regular intervals determined by the manufacturer of the equipment concerned. Defective or poorly maintained fire fighting equipment shall be removed from Site immediately and replaced accordingly. An inspection logbook must be kept and available for control by the Employer’s Safety Officer.

The public fire brigade, factory fire brigade and fire protection facilities must not be obstructed at any time during the course of the Contract. Fire hydrants shall be kept clear of any obstructions.

Non controlled open fires are forbidden.

Burning of any kind of wastes or construction materials is forbidden.

Suitable fire extinguishers shall be at hand when cutting or welding. Anyone using a fire extinguisher is responsible to get it refilled. Flammable materials shall be removed from welding, grinding or cutting working zones. A special attention shall be brought to areas located at inferior levels from these zones.

“No smoking” and “Danger-Flammable” signs shall be posted in dangerous areas.

Outbreaks of fire shall be promptly reported to the public fire brigade, the factory fire brigade and the Employer’s Site Manager.

Gas or chemical leaks shall be promptly reported to the public fire brigade, the factory fire brigade and the Employer’s Site Manager.

### First aid facilities

We will  provide and maintain adequate first aid facilities and material, as well as, if required by local laws and regulations, medical facilities. The first aid shall be available when we are working on site, at any time. We will make the necessary arrangements for emergency evacuation to the nearest hospital.

The material shall be available in sufficient quantities and easily accessible and stored in such a way to be protected against harmful effects.

A normal stretcher and a stretcher fit for rescuing inside cyclones or silos must be kept inside the Contractor office. Method statements shall be prepared for evacuation from silos and cyclones.

The completeness of the First Aid boxes shall be checked weekly by our Safety Manager and restocked immediately when necessary.

First Aid shall be readily available at any time during working hours, including shift work.

Prior the beginning of the works, special arrangements shall be made with local facilities for evacuation and medical treatment in case of injury.

The Contractor shall provide fully trained first aid personnel as required by local regulations and at least one per shift.

We will maintain a logbook of all injuries and keep it available for consultation by the Employer’s Safety Officer.

### Safety training

we conduct the necessary trainings.

Training will take the form of induction training for each employee new on the site, specific training and tool box meetings (on the job training).

The Contractor’s Safety Manager, who will have been previously informed by the Employer’s Safety Officer of the plant requirements, will give the induction training.

No access to site is allowed without this training.

It will encompass the following items:

- specifics of the plant and the plant industrial hazards

- alarm, site evacuation, personnel counting, resumption of work

- site safety rules and procedures

- local laws, rules and regulations

- contractual stipulations

- near miss and accidents reporting

- fire protection

- housekeeping

- disciplinary action policy

- etc.

This induction training must be given to all Contractors, subcontractors, sub-subcontractors and suppliers’ managers, employees and workers, as well as to visitors and any Contractor related third party, if they are not accompanied by Contractor’s personnel when visiting the site.

Attendance to this training shall be documented in a special logbook with the name of the person, the date of the training and the signature of the trainee.

Our  Safety Manager shall keep this list. The Employer’s Safety Officer shall be authorized to control it and to get copies at any time during working hours.

We will ensure that any other specific training is provided if and when required for its personnel. This may include, but is not limited to, erecting and dismantling scaffolds, works at heights, work in confined spaces, etc.

### Other General Safety Aspects

- Those engaged in welding and cutting works shall use protective goggles and hand gloves

- Wherever personnel are employed on the work of lead painting, overalls and suitable face masks shall be provided for use by the workers when paint is applied in the form of spray or when a surface having dry lead paint is being rubbed or scraped.

- We will  use only tested equipment, tools, chains, ropes, etc, and shall periodically test them to ensure good working condition of such equipment, tools, chains, ropes, etc.

- Use of cranes and hoists including their attachments, anchorage and supports shall conform to the following standard conditions:

- Cranes and hoists shall be of good mechanical construction, sound materials and adequate strength and shall be kept in good working orders.

- Every rope used in hoisting or lowering materials or as means of suspension shall be of durable quality and adequate strength and free from defects. Frayed ropes shall not be used.

- Every crane driver or hoist operator shall be properly trained and shall be conversant with safety regulations for the use of such equipment and rigging.

- All cranes and hoists shall be clearly marked with the safe working load.

- Motors, gearing, transmissions, electric wiring and other dangerous parts of hoists shall be provided with efficient safeguards.

- Rigging shall be by such means as to prevent the accidental falling of the suspended load.

# Site Practices and Conditions

## Generalities

The following rules and practice have to be followed by anyone entering the Site whatever the duration of his/her stay.

These rules must be read concurrently with the Safety and Health rules.

These rules are enforced from site take over to Provisional Acceptance.

The Contractor is responsible to have all his employees, subcontractors, sub-subcontractors, suppliers, visitors and any party with whom he has a direct or indirect contract to follow this article.

## Fencing

## The temporary fencing of the new plant will be the responsibility of the we  protect the Supplies and Contractor’s Equipment.

All normal personnel and vehicles access to the site will be through controlled gates. The we may provide a full time watch service at these selected gates at his will.

We will  be responsible for receiving and registering all his incoming equipment, materials and tools.

All safety site rules must be permanently adhered to within the fenced areas.

We make the necessary arrangements to allow quick and permanent access to the emergency and fire fighting crews and vehicles.

## Site access and circulation

Permanent access:

- Selected Contractor’s and Employer’s vehicles will be delivered permanent site entry permit. Temporary access:

- Visitors will be delivered color-coded badges.

All people entering the site must follow the safety rules including wearing PPE. Drivers, when out of their vehicle, shall comply as well. We will foresee special arrangements (availability of PPE) at the gates to ensure the implementation of this rule.

We will build and maintain all necessary site roads and pedestrian walkways.

We will ensure that the vehicles leaving the site will not pollute the plant and public roads systems. He will install, if necessary, an appropriate wheels washing station at a location approved by the Employer, and will take care of wastewater according to existing regulations.

## Speed limit and roads protection

## A maximum speed of 20 km/h will be enforced.

## Site lighting

We  will install temporary lighting for all roads, pathways and working areas. The lighting levels must obey the existing local rules (see indications in the below given table “Lighting Levels).

Lighting levels (Applicable until commissioning commences)

The contractor shall follow the local rules. As indicated values the Contractor shall provide the following lighting levels Walkways, general, platforms, accesses, stairs at all time and permanently 20 lux Areas of rough work, concreting, brickwork, rebars, piping, etc 50 lux Inside Works, compressor rooms, electrical substations 100 lux For work in confined space, 24 V lighting systems shall be used.

## Site housekeeping

At all times during we performance of the works, we are  solely and fully responsible to keep the Site and the Works free from waste materials or rubbish resulting from the Works or caused by any of the our employees, subcontractors, sub-subcontractors or suppliers.

The site will be daily cleaned and the waste disposed of in appropriate containers. The containers will be emptied as soon as they are full.

We will keep the site free from standing water.

They shall, as well, take all precautions to ensure safe operations during winter months/rainy seasons (salt or gravel sprinkling of all walkways and working platforms if necessary).

Ground and surface run-off water from sit areas shall be collected and channeled into drains and ditches which have to be designed and provided by our enjineer.

No burning of any rubbish or materials is allowed on site.

No protruding nails will be left on planks; they must be hammered down as soon as the planks are dismantled.

At the completion of the Works We will clear away, remove from the site and/or

All excavated materials, waste, rubbish, etc., have to be disposed according to the existing regulations to a licensed dumpsite.

## Site guarding

We will  provide a full time watch service for the Works, and all his facilities including but not limited to offices, office furniture and equipment, tools, materials, vehicles and equipment.

Warehousing shall be such as to prevent storage of the equipment or materials in water or mud, suitable backfill and/or drainage shall be foreseen.

Electrical or fragile equipment shall be protected according to manufacturer’s instructions.

Storage areas shall be organized in such a way as to facilitate easy and speedy retrieval of equipment, parts, tools, fixtures, accessories and materials and as not to hinder progress of work or to be a safety hazard.

## Coordination with other contractors  其他承包商之间的协调

We will  not impede site access, access to work areas and designated storage locations for other contractors or Employer. Within the scope of his work, each of our Contractor has the obligation to cooperate with scheduling of the works of other contractor(s) as may be required.

In such a case, every contractor will be allocated a designated area within the fence to set up his facilities. Such designated areas and the Works site(s) are the only locations to be occupied by contractors’ personnel.

## Preservation of Employer property

We will be responsible for any damages to property, plant or equipment used by or adjacent to the Contractor’s Work including but not limited to such plant and equipment that the Employer’s Site Manager has granted permission to the Contractor to use.

We will prevent any property and facility, including but not limited to public roads, buildings, bridges, electric or telephone cables, drains, sewers or water supplies from being damaged or injury caused to persons by any unloading, loading or traffic of the Contractor.

Where and to the extent that damages to plant, equipment or property or where injury to persons has occurred, We will notify the Employer’s Site Manager immediately as to the extent of such damage or injury and shall take all necessary and appropriate steps required to minimize further damage or injury occurring. All costs in making good, repairing or replacing damages or property, or medical and hospital fees as a result of injuries or repatriation of injured persons shall be to the account of the Contractor.

## Ammunitions, drugs, weapons, explosives Any ammunition, explosives, weapons or the like is not authorized on site. Any ammunition, explosive, weapons or the like found on site shall be immediately reported to the Employer’s Site Manager and the case where ammunition or explosives have been found, the area or areas affected shall be cordoned off to prevent access by any person. All personnel shall leave the area immediately.

The consumption, possession and sale of alcoholic beverages, drugs or any banned or illegal substances or any person under the influence of same or the carrying of weapons of any kind in, on or around the Site is strictly forbidden.

The Site security shall have the right at any time to search the Contractor’s vehicles entering the Site and can refuse admission to anyone found in possession of the aforementioned items or if any person is found to be under the influence of alcohol or drugs. Any person found to be under the influence or in possession of alcohol or drugs, or in possession of a weapon whilst on the Site shall be dismissed from the Site immediately. The cost of replacing any offending person or persons under the control of the Contractor shall be to the account of the Contractor.

## Working hours   We will follow the local laws. Night, week-end and holidays work, within the local regulatory framework, is authorized after informing the Employer’s Site Manager and under the condition that an empowered manager from the Contractor is on site during this work.

We willl take all necessary measures to reduce the noise and the disturbances when working at night.

## Fossils and antiquities  化石和古董

All fossils, coins, articles of values or antiquities, or any other structure or item of archaeological interest found within the site shall be placed under the care and authority of the Employer. The Contractor shall take all necessary precautions to prevent his personnel, his subcontractors’ personnel and all other persons from removing or damaging any of these finding.

We will, upon discovery of any such finding, promptly inform the Employer.

The Employer shall issue instructions to the Contractor specifying the ways to deal with it.

## Noise level

Above a noise level `A` (as defined in local standards) of 85dB (A) the personnel shall bear a noise protection equipment.

In no case shall the personnel be exposed to more than 105 dB (A).