TERMS OF REFERENCE FOR THE ENVIRONMENTAL IMPACT ASSESSMENT FOR THE MOUNT ROSSER BYPASS LINSTEAD TO MONEAGUE

DATE:			2007 June

CLIENT:			Bouygues Travaux Public

(Jamaican Branch)

###### TABLE OF CONTENT

0.0	INTRODUCTION

1.0	PROJECT BACKGROUND

2.0	PROJECT DESCRIPTION

3.0	PROJECT LOCATION

4.0	TERMS OF REFERENCE

- INTRODUCTION

EnviroPlanners Limited has been contracted by Bouygues Travaux Publics Limited to conduct an Environmental Impact Assessment for the construction of the Mount Rosser Bypass from Linsrted to Moneague. In keeping with the requirements of the National Environment & Planning Agency (NEPA), EnviroPlanners limited is proposing the following Terms of Reference. This is based on NEPA’s draft generic Terms Of Reference.

- PROJECT  BACKGROUND

The Highway 2000 Project (H2K) began in 2001, when the Prime Minister of Jamaica announced that Bouygues Travaux Publics of France was selected as the preferred bidder for Phase 1 of the Project.  The upgrade of the Old Harbour Bypass was the first phase of the project and was followed by the construction of the Kingston to Bushy Park segment, and then the Portmore Causeway segment.

The H2K Project will link Kingston to Montego Bay through the parishes of Kingston & St. Andrew, St. Catherine, Clarendon, Manchester, St. Elizabeth, Westmoreland and St. James. The Highway will also have a North-South link, originally intended to connect Bushy Park and Ocho Rios, traversing the parishes of St. Catherine and St. Ann. The total length of the highway is approximately 230 km, with an anticipated right-of-way of 100m.

In 1996, a “Feasibility Study and final design for upgrading Highway A1 from Spanish Town to Moneague and St. Ann’s Bay” was commissioned by the Government of Jamaica (overseen by the then Ministry of Local Government & Works) and conducted by Société Nouvelle INGEROUTE.

At that time, and as now, the existing road was not acceptable for the large traffic flow and there are a number of specific problems that reduce the traffic capacity severely.  These include the following:

- Traffic may be totally interrupted for a few hours to a few days in the Bog Walk Gorge, when the Rio Cobre is in flood,

- The Flat Bridge (a one-lane bridge), in the Bog Walk Gorge, creates a permanent bottleneck,

- Very long delays can be experienced between Ewarton and Moneague in the case of accidents or any blockage of the road, and

- On the same section, the crossing of Mount Diablo is very difficult for loaded trucks.

The study was split into two major stages.  Phase I involved a preliminary identification of alternative routes and a comparison of alternatives to identify the best option.  This ‘best option’ was determined by evaluating the technical, environmental and economical aspects of the alternatives.  Phase II focused on the preliminary engineering design of the selected ‘best option’.

In 1999, a Strategic Environmental Assessment (SEA) of the Highway 2000 Project was commissioned by the National Development Bank of Jamaica Ltd., as part of the development of the functional planning design.  The SEA report provided guidelines for design and alignment within the context of a 1 km corridor, which had been selected by the "least constraining methodology".  The report recommended areas for further study in detailed project-level environmental impact assessments (EIAs) that would be necessary to support the application for an environmental permit.  The current alignment of the North-South link does not follow the route put forward in this SEA.

TransJamaican Highway Ltd., a specially created subsidiary of Bouygues Travaux Publics, is the developer responsible for the implementation of the project, and Bouygues Travaux Publics Jamaican Branch is the contractor.

Segment I of the Highway 2000 Project has been divided into two phases as follows:

- Phase IA will include extensive work on the Portmore Causeway as well as work from Kingston to Sandy Bay.  This will include construction of four lanes from Kingston to Bushy Park (2x2) and doubling of the Old Harbour Bypass (by construction of a second 2-lane carriageway.

- Phase IB will include construction of the highway from Sandy Bay to Williamsfield, incorporating the existing Melrose Bypass, construction of the May Pen interchange and a main toll plaza at Williamsfield.

The new alignment for the North-South link closely follows the ‘best option’ as put forward by the Société Nouvelle INGEROUTE study.  As stated previously, this link was originally intended to link Bushy Park and Ocho Rios but now would incorporate use of some existing road (non-tolled sections) and the creation of new roads (tolled sections).  This will include:

- Spanish Town (tie-in with Highway 2000) to Bog Walk, bypassing the Bog Walk Gorge, (Tolled section)

- Bog Walk to Linstead (Free section)

- The Mount Rosser Bypass, from Linstead to Moneague (Tolled section)

- Moneague to Golden Grove (Free section)

- Golden Grove to Ocho Rios (Tolled section)

The Mount Rosser Bypass is one section of this North-South link, which will be merged with the entire alignment in due course.  This segment is seen as an urgent need, and has therefore been given priority.

- PROJECT DESCRIPTION

# 2.1	Alignment, Crossings and Toll Plaza

2.1.1	Aligment

The Linstead to Moneague segment of Highway 2000 (H2K) requires the construction of a two lane, dual carriageway all-purpose road that is essentially rural in nature.   The Highway will run to the east of the current A1 Highway and will cover a distance of 23.9 km.

The Highway begins with an interchange at the Linstead Bypass, St. Catherine at chainage 0+000, near Byndloss.  The alignment travels through the districts of (Stirling Castle), Russell Pen, Treadways and Palm, following a similar alignment to that of the existing rural road.

The highway then heads easterly, through Mountain Pass and towards the district of Enfield, running through agricultural land before curving towards Mullock.  From here, the highway traverses rolling or irregular terrain of Mount Zion, Schwallenburgh and Faith’s Pen, most of which had been mined previously for bauxite.

From Km 19+000, the highway follows a similar alignment to the existing A1 Highway, passing on the eastern side.  The highway then crosses the A1 Highway at the Unity Valley Farms, passing near Green Haven and Grier House, and then reconnecting with the A1 Highway, behind Moneague, on the segment to Golden Grove.

## 2.1.2	Crossings

#### The following table gives information on the structures associated with the highway.

| Location | Bridge Type | Overpass/Underpass | Structure Type |
| --- | --- | --- | --- |
| Km 0+930 | Field Connector | Overpass | Composite |
| Km 1+620 | Field Connector | Underpass | Concrete Box |
| Km 3+340 | River Crossing | Underpass | Concrete Box |
| Km 3+950 | Local Road | Underpass | Concrete Box |
| Km 5+5100 | Field Connector | Overpass | Composite |
| Km 5+600 | River Crossing | Underpass | Concrete Box |
| Km 5+700 | Field Connector | Overpass | Composite |
| Km 16+500 | Field Connector | Underpass | Concrete Box |
| Km 17+700 | WINDALCO Road | Underpass | Concrete Frame |
| Km 18+450 | Field Connector | Overpass | Composite |
| Km 21+600 | Local Road | Underpass | Concrete Box |
| Km 22+850 | Local Road | Underpass | Concrete Box |
| Km 23+400 | Field Connector | Underpass | Concrete Box |

## 2.1.3	Toll Plaza & Equipment

In order to facilitate operation as a toll road, fibre optic cables will be laid along the entire length of the highway.  The toll plaza is expected to have seven lanes.  The proposed toll plaza building will be similar to that of the Vineyards Toll Plaza, and the canopy and equipment will be similar to that of the Spanish Town Plaza.

It should be noted that lighting along the highway would only be in the vicinity of the Toll Plaza and the interchanges.

# 2.2. Phasing and Timetable

The project is scheduled to be concluded in 30 months.  The project has been divided into the following phases to suit the programme of work and acquisition of land:

Phase 1:	Km 10+000 to 18+000

Phase 2:	Km 18+000 to 22+000

Phase 3:	Km 22+000 to 23+890

Phase 4:	Km 5+000 to 10+000

Phase 5:	Km 0+000 to 5+000.

# 2.3	 Construction camp/Site Yard

The construction camp will be located in the Palms community, in the vicinity of the field connector at Km 5+100.  The camp will be approximately 200m x 200m and will take into consideration storm water and surface water drainage requirements, location of interceptors, as well as wastewater and sewage requirements.

# 2.4	Cut & Fill

All fill materials will be obtained mainly from the cut and transported by trucks to the designated fill areas.

Quarries will be identified based on:

- Proximity to project,

- Material type required

- Type of approval from authorities.

Should BYTPJ need to establish a quarry, the necessary licenses/approvals will be sought.

## 2.4.1	 Transportation requirements

All motorized vehicles within the site, excluding those on public roads, shall be restricted to a maximum speed of 20 km per hour (in the site yard) and 50 km per hour (on the alignment) by putting up speed limit signage at appropriate locations.

Haulage and delivery vehicles shall be confined to designated roadways inside the site.

The Production Team shall ensure that vehicles transporting dusty materials are fitted with side and tailboards. Materials transported by vehicles shall be covered, with the cover properly secured and extended over the edges of the side and tailboards.

Dusty materials shall be dampened before transportation.

# 2.5	 Waste Management

All waste materials will be managed in accordance with the project environmental management plan in a manner that will promote waste avoidance and minimisation.  Waste materials will be disposed of in accordance with the relevant laws, guidelines and best practices.

Waste management options can be categorised in terms of preference from an environmental viewpoint whereby the more preferable options have the least impacts and provide for enhanced sustainability.  A waste management hierarchy shall be applied on site as follows:

- Avoidance and minimisation, for example by:

  - Selecting products that will cause no or minimal environmental impacts

  - Not generating waste, which would be achieved by changing or improving practices and design;

- Reuse of materials, thus avoiding disposal; and

- Treatment and disposal, in accordance with relevant laws, guidelines and best practice.

## 2.5.1	Chemical Wastes

Paint residues, lubricants and other oily wastes will be classified as chemical wastes and special controls will be imposed to regulate storage, labeling, transport and disposal.

## 2.5.2	Construction Waste

Waste material may include any excavation spoil, sewage, waste water or effluent containing sand, cement, silt or any other suspended or dissolved material to flow from the site onto any adjoining land, storm drain, sanitary sewer, or any waste matter or refuse to be deposited anywhere within the site or onto any adjoining land.

When handling the waste material, the following measures shall be undertaken:

- The strategy for management and disposal of all wastes arising from the project will be based on the principle of segregation and re-use on site followed by disposal at Parish Council approved reclamation, public dump or landfills as appropriate;

- Surplus excavated material is to be exported to a Parish Council approved public dumping areas;

- All construction waste shall be sorted on site into inert and non-inert materials.  Non-inert materials such as wood and other materials including glass, plastics, steel and metals shall be disposed of to landfill.  Inert materials like soil, sand, rubble, shall be separated from non-inert material and disposed of at Parish Council approved public dumps or spoil sites

- For disposal of other inert construction wastes, principles shall be established to:

  - Maximise re-use of materials on site,

  - Segregate wastes to ensure that the minimum quantities are disposed of at Parish Council approved landfills,

  - Maximise reclamation of materials by the public.

- In addition, quantities of site fencing and timber for the building work shall be reused where possible. Those materials that cannot be re-used will require disposal at a Parish Council approved landfill;

- All vehicles carrying waste shall have properly fitted side- and tailboards, and the materials being transported shall be securely covered.

## 2.5.3	General (Non Construction) Waste

The following principles shall be adopted for General (Non Construction) Waste:

- All works areas shall be cleaned of general litter and refuse daily.

- General refuse and litter shall be stored in enclosed bins or compaction units separate from construction or chemical wastes. A reputable waste collector shall be used to remove general waste and litter off site for disposal.

- Refuse shall not be burned at any Construction Area.

- General refuse may be generated by food service activities on site, so reusable rather than disposable dishware shall be used if feasible.

- Office wastes shall be reduced through recycling of paper, if volumes are large enough to warrant collection. Participation in a local collection scheme shall be considered, if available.

3.0	PROJECT LOCATION

Figure 1. show a map detailing the location of the project (orange line).

4.0	TERMS OF REFERENCE

The Environmental Impact Assessment to be completed by by EnviroPlanners limited shall include but not limited to the following:

- Objectives of the project

- A complete description of the existing site proposed for development outlining the main elements of the project with particular emphasis on the details of the highways as it relates to the number of lanes as well as the plans for future expansions. A map will be included to identify the proposed location of the project site.

- Significant environmental issues of concern will be identified through presentation of baseline data which will take into consideration social, cultural and heritage information. An assessment of the public perception of the proposed development will be done through public consultations and the use of social survey instruments such as questionnaires.

- Identification of Policies, Legislation and Regulations relevant to the project.

- Prediction of the likely short, medium and long term impact of the development on the environment, including direct, indirect and cumulative impacts, and their relative importance to the design of the development’s facilities.

- Identify any mitigation action to be taken to minimize predicted adverse impacts and provide associated costs where applicable and practical.

- Develop an Environmental monitoring Plan which will ensure that the mitigation measures are adhered to during the implementation phase.

- Describe the alternatives to the project.

- Conclusions

EnviroPlanners Limited will provide full details of the following in fulfillment of an Environmental impact Assessment:

- Task # 1.	Description of the Project

  - Detailed description of the area proposed for the project

  - Detailed description of the elements of the projects such as the alignment, crossings, bridges and toll plaza

  - Description of proposed off site facilities such as construction camps and infrastructure service

  - Description of project related decommissioning and abandonment of works and/or facilities.

  - Information on location, general layout and size of project area.

  - Use of maps, site plans, photographs and graphics aids where necessary.

  - Description of construction, pre-construction and post construction activities

2.	Task # 2.	Description of the Environment/Baseline Studies Data Collection and Interpretation

Presentation of baseline data which will be utilized to describe the study areas in respect of the following:

- Physical

  - Detailed description of the existing geology and hydrology. Determination of storm water run-offs and the effect of the project on the ground water. Slope stability issues will be explored.

  - Water quality of any existing rivers, pond or streams in the vicinity or to be impacted by the development. Water Quality Indicators should be but not limited to suspended solids, colour, turbidity, nitrates, phosphates, oil & grease and faecal coliform.

  - Climatic conditions and air quality in the area of influence including particulate emissions from stationary or mobile sources, NOx, Sox, wind speed and direction, precipitation, relative humidity and ambient temperatures,

  - Noise levels of undeveloped site and the ambient noise in the area of influence.

  - Obvious sources of existing pollution and extent of contamination.

  - Availability of solid waste management facilities.

- Biological

  - Description of flora and fauna in the sphere of influence of the proposed project with emphasis on rare, threatened, endemic, protected, endangered species.

  - Description of the different ecosystem types (e.g. caves and sinkholes if present) and the species they support.

  - Identification of any nocturnal species as well as migratory species which use the project site

  - Species dependence, habitat/niche specificity, community structure and diversity will be considered.

  - Note the importance of the biological diversity of the area

  - Note the invasive and economically important species.

- Socio-economic & Cultural

  - Present and projected population

  - Present and proposed land use

  - Planned developmental activities

  - Identification and summary of private land acquisition

  - Local economic benefits and cost overall and on an individual community basis

  - Implications of the project during the construction phase for resident commuter travel and travel times

  - Accommodation for construction workers

  - Access to, and delivery of health, educational and social services and emergency support to local communities

  - Employment

  - Distribution of income, goods and services

  - Recreation

  - Public heath and safety

  - Historical importance of the area

  - Public perception

Task #  3.	Legislative & Regulatory Considerations

Describe the pertinent legislation, regulations and standards, and environmental policies that are relevant and applicable to the proposed project, and identify the appropriate authority jurisdictions that will specifically apply to the project, so as to ensure compliance.

There are several pieces of legislation, regulation and policies which are applicable to developments of this nature. Some that will be examined are; the Toll Road Act, Wild Life Protection Act, NRCA Act, Public Health Act, and the Town and Country Planning Act.

Task # 4.	Determine the Potential Impact

Identify impacts related to highway construction activities, such as land clearing, land cutting spoil disposal and possible land filling. Distinguish between significant impacts that are positive and negative, direct and indirect and short and long term. Identify impacts that are cumulative, avoidable or irreversible. Identify any information gaps and evaluate their importance for decision-making. Special attention will be paid to the effect of the project on:

- Water quality - surface and underground

- Drainage pattern and flooding

- Waste disposal

- Changes to land use

- Future development

- Geological, archaeological, historical, architectural and any other naturally occurring features/resources

- Atmospheric pollution

- Biodiversity and ecosystem

- Socio-economic and cultural

- Habitat loss or disturbance

- Noise levels ,

Task # 5.	Drainage Assessment

Storm water run-off will be examined and assessed on the basis of;

- Run-off during construction and the necessary mitigation for flooding and sedimentation of water bodies.

- Rivers, gullies and other wetland crossing and potential impact of the construction and constructed features on the water bodies.

- Run-off during operation and the necessary mitigation for flooding and sedimentation of water bodies.

Task # 6.	Mitigation & Management of Potential Impacts

Identify possible measures to prevent or reduce significant negative impacts to acceptable levels with particular attention paid to drainage and dispersal/sedimentation control, as well as measures to minimize disruption to existing fauna and flora. Cost the mitigation measures, equipment and resources required to implement those measures. Propose mechanisms for investigating claims for compensation put forward by affected stakeholders.

Task # 7.	Monitoring Plan

Identify the critical issues and features requiring monitoring to ensure compliance to mitigation measures and develop an Environmental Monitoring Programme indicating the parameters to be monitored, the recommended frequency and duration of the monitoring.

Task # 8.	Analysis of Alternatives to the proposed Project

Describe the alternatives examined for the proposed project that would achieve the same objective including the “no action alternative. This includes consideration of historical use of the area and future planned developments where applicable.

Task # 9.	Public Consultation & Presentation

Stakeholder groups such of Environmental Organizations NGO & CBO will be identified and consulted during the period of conducting the EIA. Consultation will be through surveys, interviews and meetings.

A public Presentation of the EIA report will be done to inform and seek discussion and comments from the public on the proposed project. The implementation of the project will span two parishes, St. Catherine and St. Ann. Two Public Meeting are therefore proposed for Linstead and Moneague