DRAFT

ENVIRONMENTAL MONITORING PLAN

MOUNT ROSSER BYPASS LINSTEAD TO MONEAGUE

Between markers 0+000 km – 10+000  and 18+ 500 km – 24+000

DATE:			2007 August 24

CLIENT:			Bouygues Travaux Public

(Jamaican Branch)

###### TABLE OF CONTENT

1.0	OBJECTIVE

2.0	FLORA)

3.0	FAUNA

4.0	MONTORING PARAMETERS

- MONITORING FREQUENCY

- REPORTING

- MONITORING CHECKLIST

- OBJECTIVE

The objective of the Environmental Monitoring Plan (EMP) is to ensure that construction activities are carried out in a manner that minimizes the negative impact on the environment. And that there is full compliance with all the conditions stated in the permit. The monitoring programme will pay special attention to all conditions set out in the permit to ensure compliance.

- FLORA

Prior to the start of construction a detailed assessment of the Flora within the footprint of the highway is recommended. All endemic, endangered, rare, threatened, or ecologically valuable plant species found within the footprint of the highway should be identified and tagged for relocation to the nursery.  A nursery should be established to accommodate plant identified for relocation.

Fortnightly checks in the first two (2) months of construction and monthly checks

thereafter until construction is completed should be conducted on the endemics and other

trees that are relocated to the nursery to ascertain their conditions. In addition, there should be checks to ensure that the plants are replanted at an appropriate location within the project area and their health noted.

3.0	FAUNA

Prior to the start of construction a detailed assessment of the Fauna within the footprint of the highway is recommended. All endemic and endangered species encountered should be relocated within the area, and if injured sent to a Rescue Centre approved by the Authority. Written consent from a Rescue Centre to care for any injured endemic or endangered fauna found within the project area should be obtained.

4.0	MONITORING PARAMETERS

Fortnightly checks in the first two (2) months of construction and monthly checks until the completion of construction should be done on the following;

1. 	Inspection of vegetation clearance activities to ensure that it is being done 			on a phased basis in tandem with the phase development of the highway.

2. 	Monitoring to ensure that air quality are maintained within NRCA 				Ambient Air Quality Regulation of 1996.

3. 	Inspections of trucks carrying particulate material to ensure that they are 	  	   	 properly covered to prevent entrainment of particulates in the ambient air.

4. 	Monitoring to ensure that noise levels do not exceed 70dB at the property 	    	   	 boundary at any given time.

5. 	Inspection of stockpiles and unpaved roadway to ensure that measures 			such as wetting is done to minimize fugitive dust emission during the 			construction phase.

6.	Assess the availability of portable sanitary chemical toilets to satisfy 			workers needs during construction activities.

7. 	Assessment of the disposal of solid waste generated during the 				construction of the highway to ensure the disposal is done at a municipal 			dumpsite with the approval of the National Solid Waste Management 			Authority.

9. 	Inspection of quarry licenses, if applicable, to ensure that they are legal, 			and copy of licenses are submitted to the NEPA.

10	Inspection of stockpiles to ensure that they are contained to prevent 			entrainment by runoff during rainfall events.

11.	Inspection to ensure that there is no discharge of oil, oily waste, trade or 			sewage effluent chemicals solid waste, poisonous noxious or polluting 			substances is discharged on or into the ground without a license from the

NRCA.

12.	Check on blasting activities to ensure that blasting or use of explosives is 			done with the relevant written approval from the Mines and Geology 			Division.

13. 	Check on the discovery of any artifact, archeological, cultural, economic or 			ecological features unique specimens or significant deposit of fossil.

5.0	MONITORING FREQUENCY

It is proposed that the duration and frequency of each monitoring exercises will be adjusted as the construction area and activities increase to ensure adequate scrutiny of all relevant areas and activities to be monitored.

6.0	REPORTING

All non- conformances and any other urgent or critical issues should be reported to the project manager or designate immediately at the time of discovery during the site inspection visit. A full monitoring report should be compiled and submitted to the project manager within seven days of each monitoring event.

A monthly report on the monitoring activities is recommended. It is proposed that the reports cover periods of a full calendar month and become due within seven days of the end of the month.

7.0	MONITORING CHECKLIST

| QUESTION | YES | NO | COMMENTS |
| --- | --- | --- | --- |
| Detail assessment of Flora & Fauna done before construction started |  |  |  |
| Endangered & endemic species identified & relocated |  |  |  |
| Are plant in the nursery healthy |  |  |  |
| Any injured Endangered on endemic fauna encountered |  |  |  |
| Is vegetation clearing activities in tandem with phase development |  |  |  |
| Is ambient air quality within regulation |  |  |  |
| Is the maximum noise levels below 70dB |  |  |  |
| Are trucks transporting Particulate material adequately covered. |  |  |  |
| Are measures such as wetting in place to minimize fugitive dust from stockpiles |  |  |  |
| Are containment measures in place to prevent entrainment of stockpile material during rainfall |  |  |  |
| Are adequate portable chemical toilets in place to satisfy workers needs |  |  |  |
| Is solid waste being disposed at the approved municipal dumpsite |  |  |  |
| Is license in place for quarrying activities where applicable |  |  |  |
| Is there discharge of chemicals oil poisonous or noxious substances |  |  |  |
| Does any blasting activity have the relevant approval |  |  |  |
| Was any artifact or other significant feature encountered |  |  |  |