APPENDIX III

Table A.1  	Potential Impacts and Mitigation Measures for Soils and Slopes - Construction Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Clearing of forests & woodlands Blasting & stripping Cutting | Loss of productive soils (topsoil). | Long Term | Where possible, remove topsoil and store it where landowners may come and collect it or spread it over existing adjacent agricultural land or reuse it to restore affected areas Limit all work activities within the right of way of the Highway. |
| Movement of heavy machinery Blasting & stripping | Erosion of surface soils and modification of slope stability Increased risk of landslide and loss of productive soil. | Long Term | Avoid crossing known unstable areas. If erosion is likely, build ditches, barrier or basins to allow eroded soils to precipitate during rains. Ensure installation of Gabions or other types of retention structures to limit erosion and increase slope stability. If the existing vegetation is removed, plant a new vegetation cover of endemic or indigenous species. Select fast growing species that are characterized by a dense and intricate root system. During the land clearing phase, keep the root system in place to help stabilise the surface soil in steep areas. |

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Movement of heavy machinery Clearing of land | Compaction of soils. Modification of surface drainage and of water infiltration. Reduces growth rate of cultivated plants. | Medium Term | Limit all work activities within the right of way of the Highway. Where soils have low bearing capacity, use vehicles with caterpillar bands or that apply low pressure on soils. At the end of construction work, measures will be taken to restore surrounding soils to their natural state using a scarifier or similar equipment. |
| Cut and fill. | Modifications of topography | Long Term | Avoid or minimise cut and fill in areas with recognised aesthetic values. |
| Cut and fill. | Increase erosion potential and landslide sensitivity. | Long Term | Minimise the use of dynamite or other explosives for cutting into the rock stratum. Include slope netting or other stabilizing Reforest slopes where possible. |
| Illegal dumping of material excavated on the hillsides. | Strong natural habitat modification | Long Term | Ensure construction activities are limited to the right-of-way. Prevent side-tipping of spoils. |
| Opening new vistas or filling small valleys | Perturbation of landscape | Long Term | Plan the landscaping of the cut and filled areas to improve the quality of the views that are modified |
| Side tipping of spoils and fills. | Compaction of soils. Loose and uncontained soils can wash away Generation of dust | Medium Term | Under no circumstances can dumping of excavated material be allowed on the sides of the right of way. Financial penalties should be imposed on contractors that do not conform. All excavated material must be used for filling needs or stored in approved and clearly identified sites. |
| Cutting & Clearing | Drainage modification Local flooding and long term water accumulation. | Long Term | Ensure surface water flow at all time. Build temporary ditches to redirect interrupted water flows. Depressions created by machinery and truck movement must be filled. |
| Use of chemicals Use of paints Use of herbicides/ pesticides Accidental spills Use of combustion equipment | Contamination of soils. Contamination of surface and groundwater. | Medium Term | - Use only vehicles that are in good working condition, and proceed rapidly with all repairs needed to avoid leakage of contaminant. - All contaminants must be stored on adequately designed sites equipped with spill intervention kits. The storage sites must be located outside of known flood plains and away from landslide areas. - When filling vehicles with potential contaminants such as diesel, gas, oil and lubricant, install an absorbent mattress under the vehicle. Keep a spill intervention kit at the filling site. |

Table A.2	 Hydrology and Water Resources – Site Preparation/Construction Phase

| #### Activity | Negative Impacts |  | Duration | Mitigation Measures |  |
| --- | --- | --- | --- | --- | --- |
| Deforestation/ removal of soil cover, especially on slopes important to watershed management. | Increased soil erosion – sheet wash, gullying and headward erosion. -Increased suspended sediment in streams -Increased flood peaks -Increased incidences of flash flooding. -Ecological effects – sedimentation of riparian and aquatic habitats. |  | Short term, but could become long term if slopes are not reforested | Avoid activity during wet season Reforest or encourage soil cover as soon as possible after clearing Install check dams, gabions and other erosion control measures. Ensure maintenance of riparian vegetation |  |
| Exposure of bedrock | Sediment loading and resulting effects |  | Short term | Avoid activity during wet season |  |
| Construction activities – cutting, blasting, earth moving, filling, bridge building. | Sediment loading. Vast increases in sediment load to rivers -Increases in water treatment costs -Aggravation of stream bed -Increased scour during floods -Ecological effects – sedimentation of riparian and aquatic habitats |  | Short term | Avoid activity during wet season Terracing of slopes Slope stabilization Install check dams, gabions and other erosion control measures. Storage of excessive rainfall/runoff e.g. check dams Settling ponds/dams to trap sediment Prohibit use of gully/stream/river as disposal sites Establish vegetation to stabilize stream banks to at as buffer to riparian zone. |  |
| Gravel mining (source of building material) | Multiple impacts in streams - increased bank erosion -channel deepening -increases in suspended sediment |  | Short term | Identify specific reaches of rivers that may be safely mined Regulate and monitor mining activity |  |
| Excavation of slope | Lowering of local water table deterioration of vegetation loss of agricultural land limestone cavern/sink hole collapse reduced supply to wells; loss of ground water for drinking |  | Long term | Replacement of existing vegetation with vegetation that requires less water. Compensation to farmers affected. Direct road runoff to groundwater recharge |  |
|  | Positive Impact | Duration |  |  |  |
|  | #### can reduce susceptibility to flooding | #### Long term |  |  |  |

| #### Activity | Negative Impacts |  | Duration | Mitigation Measures |
| --- | --- | --- | --- | --- |
| - Creation of embankments and structures | Raising of local water table/creation of perched water table - deterioration of vegetation - increased flood risk |  | Long term | Replacement of existing vegetation Drainage diversion measures |
|  | Positive Impact | Duration |  |  |
|  | increased availability of drinking water | Long term |  |  |

| #### Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Diversion of drainage- overland | Lowering of local water table and resulting effects. Flow concentration and resulting increase in speed of flow can contribute to flooding, soil erosion, channel modification and siltation. | Long term | Flow velocity reduction measures such as grass, riprap in channels. Avoid flow concentration with cutoff drains and diversion of drainage. Diverted water can be used for irrigation |
| #### Activity | Negative Impacts | Duration | Mitigation Measures |
| Diversion of drainage - to underground | Raising of local water table or creation of perched water table. Can lead to -vegetation changes -overloading of slopes and slope failure -waterlogging of soils or flooding -Increase in ground water gradient where drainage is diverted down a sink hole – may lead to increased solution and eventual subsidence. - Groundwater recharge (+ve) | Long term | Replacement of existing vegetation Careful hydrological and slope investigation Careful drainage routing Careful hydrological investigation Monitoring |

| #### Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Restriction of drainage through culverts | Risk of flooding and/or damage to road if culverts are too small to handle seasonal and flood flow | Long term | Use empirical data to design culverts to accommodate flood flows Maintenance and clearing of culverts |
| Straightening of channels | Increases in efficiency and therefore flow velocity – could result in flooding downstream during high flows and channel modification. Channel steepening and base level lowering, resulting in channel incision upstream which can in turn increase sediment load. | Long term | Flow velocity reduction measures such as grass, riprap in channels Small check dams or gabions upstream to check erosion. |
| Heightening and strenghtening of existing levees or building of artifical levees and embarkments | Increased efficiency of river, - downstream erosion | Long term | Flow velocity reduction measures such as grass, riprap in channels |
| Emplacement of embankments. | Increased stream power if too close together | Long term | Design criteria - Set bank embankments to allow for channel migration and to accommodate floods. |

Table A.3	Hydrology and Water Resources – Operation/Maintenance Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| - Impoundment of flood waters by road | - Inundation of areas upstream of road | Long term | Design placement of structures based on careful hydrological study and use of empirical data. Design culverts under road to handle storm runoff. |
| Road may act as channel during high flows Drainage ditches may act as flood channels or chutes | Scouring of road surface Erosion/undermining of road bed | Long term | Design placement of structures based on careful hydrological study and use of empirical data Provision for diversion of stormwater runoff. |
| Flow of polluted runoff into sink holes | Pollution of aquifers and ground water supply. | Long term | Use of infiltration ditches Use of settling basins/ponds to remove pollutants before road runoff is allowed to percolate into aquifer. Oxidating macrophytes can be used to help remove the pollutants |
| Flow of polluted runoff into streams | Pollution of domestic water supply. | Long term | Use of setting basins/ponds to remove pollutants before road runoff is drained into adjacent streams. Leave a buffer zone of undisturbed riparian vegetation along river banks |

Table A.4   Sources, Health and Environmental Effects of Air Pollutants

| Pollutant | Source(s) | Health Effects | Environmental Effects |
| --- | --- | --- | --- |
| Ozone (ground level ozone is the principal component of smog) | Chemical reaction of pollutants; VOCs and NOx | Breathing problems, reduced lung function, asthma, irrates eyes, stuffy nose, reduced resistance to colds and other infections, may speed up aging of lung tissue | Ozone can damage plants and trees; smog can cause reduced visibility. Property damage - damages rubber, fabrics, etc. |
| VOCs (smog formers) | They are released from burning fuel (gasoline, oil, wood coal, natural gas,etc.), solvents, paints, glues and other products used at home and work. Cars are an important source of VOCs. VOCs include chemicals such as Benzene, Toluene, Methylene Chlorides and Methyl Chloroform. | In addition to ozone (smog) effects, many VOCs can cause serious health problems such as cancer and other effects. | Ozone smog effects, some VOCs such as Formaldehyde and Ethylene may harm plants. |
| Nitrogen Dioxide (One of the NOx; smog forming chemical) | Burning of gasoline, natural gas, coal, oil etc. Cars are an important source of NO2 . | Lung damage, illnesses of breathing passages and lungs (respiratory system). | Nitrogen Dioxide is an ingredient of acid rain (acid aerosols), which can damage trees and lakes. Acid aerosols can reduce visibility. Property damage – acid aerosols can eat away stone used on buildings, statues, monuments, etc. |

| Pollutant | Source(s) | Health Effects | Environmental Effects |
| --- | --- | --- | --- |
| Carbon Monoxide | Burning of gasoline, natural gas, coal, oil etc. | Reduces ability of blood to bring Oxygen to body cells and tissues; cells and tissues need Oxygen to work. Carbon Monoxide may be particularly hazardous to people who have heart or circulatory (blood vessel) problems and people who have damaged lungs or breathing passages. |  |
| Particulate Matter (PM 10; dust, smoke, soot) | Burning of wood, diesel and other fuels; industrial plants, agriculture (plowing, burning of fields), unpaved roads. | Nose and throat irritation, lung damage, Bronchitis, early death. | Particulates are the main source of haze that reduces visibility. Property damage – ashes, soot, smokes and dusts can dirty and discolor structures and other property, including clothes and furniture. |
| Sulphur Dioxide | Burning of coal and oil, especially high-sulphur coal from the Eastern United States; industrial processes (paper, metals) | Breathing problems, may cause permanent damage to lungs. | SO2 is an ingredient in acid rain (acid aerosols), which can damage trees and lakes. Property damage – acid aerosols can eat away stone used on buildings, statues, monuments, etc. |
| Lead | Leaded gasoline (phased out in Jamaica), paint (houses, cars), smelters (metal refineries); manufacture of lead storage batteries. | Brain and other nervous system damage; children are at special risk. Some lead-containing chemicals cause cancer in animals. Lead causes digestive and other health problems. | Lead can harm wildlife. |

Source:  Adapted from EPA- Office of Air Quality Planning and Standards

Table A.5  	Air Quality – Site Preparation/ Construction Phase

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Movement of construction equipment | Tail pipe and evaporative emissions include CO, NOx, particulate matter, CO2, Pb, CH4, N2O, Benzene etc. These emissions have the potential to cause chronic respiratory illness, cancer, headaches and premature deaths. | Short Term | Haulage vehicles and equipment should be in proper working order. All attempts should be made to use newer models. Use vehicles with established low emissions. Also ensure that vehicles are properly tuned. |
| Site clearance | Particulates (PM 10) - Has the potential to cause chronic respiratory illness, asthma attacks and premature deaths. | Medium term | Taking into account prevailing wind direction & speed, soil type, clearing only the area necessary & population densities. Access roads can be wetted or consolidated by use of binding materials. Wetting should be done approximately every three (3) hours. . In addition public relations and community consultations should be employed. |

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Transportation of construction materials and waste | Transportation load losses | Medium term | Vehicles should be properly loaded and covered. |
| Transportation of construction materials and waste | HAZMAT accidents & spills | Medium term | Emergency response plans should be in place. |
| Maintenance of construction equipment | Improper disposal of vehicular used parts and fluids such as tyres, batteries, oils, etc. | Medium term | Provide proper disposal sites and mechanisms. |
| Blasting | Generation of particulates leading to respiratory problems and reduced visibility. In addition it may impair plant metabolism. | Medium term | Put heavy mats over the area that is being blasted so as to prevent large chunks of debris from becoming airborne. In addition public relations and community consultations should be employed. Taking into account prevailing wind direction & speed, soil type, clearing only the area necessary & population densities. In addition public relations and community consultations should be employed. |

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Ground excavation | Generation of particulates leading to respiratory problems and reduced visibility. In addition it may impair plant metabolism. | Medium term | Taking into account prevailing wind direction & speed, soil type, clearing only the area necessary & population densities. In addition public relations and community consultations should be employed. Access roads can be wetted or consolidated by use of binding materials. Wetting should be done approximately every three (3) hours. |
| Cut & Fill | Generation of particulates leading to respiratory problems and reduced visibility. In addition it may impair plant metabolism. | Medium term | Taking into account prevailing wind direction & speed, soil type, clearing only the area necessary & population densities. In addition public relations and community consultations should be employed. Access roads can be wetted or consolidated by use of binding materials. Wetting should be done approximately every three (3) hours. |

Table A.6  Air Quality – Operation/Maintenance Phase

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Routine vehicular traffic | Tail pipe and evaporative emissions include CO, NOx, particulate matter, CO2, Pb, CH4, N2O, Benzene etc. These emissions have the potential to cause chronic respiratory illness, cancer, headaches and premature deaths. | Long term | Require vehicular emissions testing, legislation and enforcement. Roads should remain paved. Require that steep gradients and sharp turns be at a minimum (Design criteria). Ensure that vehicles are properly tuned. In addition speeds should be between 80 - 100 km/hr. |
| Routine vehicular traffic | Particulates (PM 10) - Has the potential to cause chronic respiratory illness, asthma attacks and premature deaths. | Long term | Require vehicular emissions testing, legislation and enforcement. Require that steep gradients and sharp turns be at a minimum. (Design criteria) |
| Routine vehicular traffic | Fugitive Dust | Long term | Require that the road remains paved. In addition vegetation maybe planted along the verges so as to act as filters |
| Routine vehicular traffic | Emissions of refrigerant agens from vehicle air conditioners CFCs and HFCs are potential greenhouse gases. | Long term | Require vehicular a/c testing to minimize leakage, legislation and enforcement. |
| Movement of industrial vehicles | Transportation load losses (eg. Bauxite and quarry vehicles). | Long term | Requires vehicles to be properly loaded and covered. Legislation and enforcement. |
| Movement of industrial vehicles | HAZMAT accidents & spills (eg. Oil trucks). | Long term | Legislation & emergency response plans. |
| Movement of routine vehicular traffic | Improper disposal of vehicular used parts and fluids such as tyres, batteries, oil, etc. | Long term | Provide proper disposal sites and mechanisms. In addition there is need for public education and awareness-raising. |
| Maintaining landscape | Herbicides/pesticides | Long term | Ensure proper usage |
| Maintenance of bridges and structures | Lead based paints for bridges | Long term | Use lead free paints. Zinc based paints maybe used. |
| Maintenance of road surface | Increase in particulate and HAZMAT | Medium term | Require that the road surface remain paved |

Table A.7	Noise – Site Preparation/Construction Phase

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Land clearing | Generation of noise above nuisance levels for long periods | Short term | Where activities occur outside of normal working hours, public relations and community consultations should be employed. |
| Blasting | Increase noise levels | Short term | Where activities occur outside of normal working hours, public relations and community consultations should be employed. |
| Ground excavation | Generation of noise above nuisance levels for long periods | Short term | Where activities occur outside of normal working hours, public relations and community consultations should be employed. |
| Cut & Fill | Generation of noise above nuisance levels for long periods | Short term | Where activities occur outside of normal working hours public relations and community consultations should be employed. |
| Movement of construction machinery | Improper driver behavior | Short term | Supervision, education and sensitizing |

Table A.8:	 Noise – Operation/Maintenance Phase

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Blasting | Generation of noise above nuisance levels for long periods | Long term | Where activities occur outside of normal working hours, public relations and community consultations should be employed. |
| Maintenance of road surface | Increased noise levels due to road surface | Long term | Use open graded asphalt, bituminous surface layers, avoidance of surface dressings & generally smooth well maintained surfaces without cracks and grooves. |
| Movement of routine vehicular traffic | Vehicular noise, increased vehicular speed increases noise levels, improper driver behaviour and disruption of community activities | Long term | Legislation, education and enforcement. Avoid steep grade and sharp corners, installation of noise barriers and maximize distance (where possible) of highway from settlements. Vegetation may also be planted along highway verges to act as noise barriers. |
| Operating and movement of maintenance equipment | Increase in noise levels | Long term | Where activities occur outside of normal working hours, public relations and community consultations should be employed. |

Table A.9	Flora -	Site Preparation/Construction Phase

| #### Activity | #### Negative Impacts | Duration | Mitigation Measure |
| --- | --- | --- | --- |
| - Clearing of forests, woodlands, cultivated areas - Cut & fill | Loss of habitat | Long Term | Where rare, threatened, endangered or protected species are affected, limit construction activities to within the Right-of-Way. Prevent side-tipping of spoils onto adjacent areas. Prevent illegal dumping of solid waste. Select proper storage sites of construction materials. |
| Clearing of forests, woodland etc | Fragmentation of habitat from splitting habitat during | Long Term | Ensure that portions of habitat remaining are viable. Land compensation to replace lost acreage. Reduce construction impacts such as dumping of cut and fill onto vegetated areas. |
| - Movement of heavy equipment - Clearing of forests & woodlands - Cut & Fill | Loss of specimens (individual trees plants) | Long Term | Construction activities should be restricted to within the right-of-way in order to reduce the number of specimens lost. Establish a Tree Preservation Order for specific specimens, including large, old trees. |
| - Movement of heavy equipment - Clearing of forests & woodlands Cut & Fill | Loss of species (For plants of limited range loss of specimen could also mean loss of the species) | Long Term | Identify rare or endemic species and red flag for preservation. Harvest rare or endemic species for translocation to other sites or for arboreta. Select proper storage sites for construction materials. Prevent side-tipping of spoils onto adjacent areas. |
| Landscaping | Introduction of exotic invasive species | Long Term | - Use endemic or indigenous species for landscaping - Secure relevant permits for introducing exotics |
| Increase in human activity due to construction workers on site | Increase risk of fires | Medium Term | Construct adequate barriers. Restrict construction activities to within the Right–of-Way Levy penalties and fines for breaches of mitigation measures |
| Clearing of forests Preparation of right-of-way | Increased access to areas for orchid poachers. | Short Term | Construct adequate barriers to prevent access to habitats from the highway. |

Table A.10:  Flora – Operation/Maintenance Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Use of highway Maintenance of toll booths Maintenance of structures | Increased access to areas for orchid poachers. | Long Term | Construct adequate barriers to prevent access to habitats from the highway. |
| Use of highway | Increase risk of fires due to increase in human activity | Long Term | Construct adequate barriers. Restrict construction activities to within the right –of-way Levy penalties and fines for breaches |

Table A.11:  Positive Impacts – Flora

###### Potential Positive Impacts It is anticipated that there will be some positive impacts. These include: - Information gathering based on required research for ‘red flag’ areas. - Build scientific data base for conservation and environmental management. - Increase knowledge of biodiversity through interpretive signs. - Preservation of specimens.

Table A.12	Fauna – Site Preparation/Construction Phase

| Activity | Negative Impacts | #### Duration | #### Mitigation Measures |
| --- | --- | --- | --- |
| Clearing of forests, woodland and cultivated areas | Loss of habitat | Long term | Limit construction activities to within the Right-of-Way. Prevent side-tipping of spoils onto adjacent areas. Prevent illegal dumping of solid waste. |
| Clearing of forests, woodland and cultivated areas | Fragmentation of habitat (birds, reptiles, amphibians, butterflies, mammals) | Long term | Ensure that portions of habitat remaining are viable, through land compensation to replace habitat lost. |
| - Clearing of forests, woodland, and cultivated areas and movement of heavy equipment - Blasting | Loss of specimens (reptiles, amphibians) invertebrates | Long term | Limit construction activities to within the Right-of-Way. Prevent side-tipping of spoils onto adjacent areas. Prevent illegal dumping of solid waste. |
| Preparation of right of way | Obstruction of wildlife routes (coneys, birds) | Medium Term | - Maintain wildlife corridors. - Design Criteria |
| - Clearing of forests, woodland, and cultivated areas and movement of heavy equipment - Blasting - Stripping | Disruption of feeding, breeding and roosting areas (bats, birds, butterflies) | Short Term | Construction activities should be conducted outside of main breeding seasons. Identify and avoid main breeding/roosting areas. Identify, flag and maintain main feeding trees. |
| Preparation of right of way | Increased access to areas for wildlife poachers. | Medium Term | Construct adequate barriers to limit access to wildlife habitats from the highway. |
| Activity | Negative Impacts | #### Duration | #### Mitigation Measures |
| - Land fill - Construction of bridges for river crossing - Exposure of top soil through land clearing | Aquatic habitat damage | Long Term | Ensure proper storage of construction material to prevent downstream siltation. Minimise rechanneling of waterways. Where rechanneling is essential try to maintain natural obstacles such as boulders Sedimentation traps. |
| - Clearing of forests, woodland, and cultivated - Cut & fill - Blasting - Stripping | Interruption of biogeochemical cycles | Medium Term | Minimise removal of biomass. Minimise relocation of topsoil. Prevent alteration of flows of surface and ground water. |
| - Blasting - Cut & fill - Laying road surface | Interruption of life cycle through loss of juveniles (caterpillars) | Medium Term | Applications of water on exposed earth and construction materials to reduce dust levels. |
| - Blasting - Stripping - Drilling | Increase in noise levels and vibration | Short Term | Adhere to consideration schedules to lessen impact |

Table A.13	Fauna - Operation/Maintenance Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| - Maintain toll booths - Maintenance of structures - Use of highway - Routine maintenance activities | Increased access for poachers (parrots) | Long Term | Construct adequate barriers to prevent access to forested areas from the highway |
| 24 hour traffic | Increase in noise levels | Long Term | Construct adequate sound barriers to minimize traffic sounds |
| Routine traffic | Road Kills | Long Term | Design criteria, Establish animal crossings underneath road. Construct adequate barriers/fences to prevent encroachment. |

Table A.14     Positive Impacts – Flora

###### Potential Positive Impacts It is anticipated that there will be some positive impacts. These include: - Information gathering based on required research for ‘red flag’ areas. - Build scientific data base for conservation and environmental management. - Increase knowledge of biodiversity through interpretive signs. - Preservation of specimens.

Table A.13	Parks and Protected  Areas – Site Preparation/Construction Phase

| Activity | Negative Impacts | #### Duration | #### Mitigation Measures |
| --- | --- | --- | --- |
| Clearing forest and woodlands | Increase unauthorized access to protected areas | Medium Term | No interchanges that will allow unauthorised access to parks and protected areas. Ensure adequate boundaries to limit access. Enforcement of regulations |
| - Clearing forests, woodlands - Land fill | Loss of ecologically sensitive areas | Long Term | Provide options for land replacement. Rehabilitation of adjacent lands Design criteria |
| - Clearing forests & woodlands - Demolition of woodlands | Loss of acreage | Long Term | Compensation by extending boundaries to maintain acreage. |
| - Construction camp activities - Clearing forests and woodland | Increased risk of fire | Medium Term | Limit construction activities to run within the right-of-way Construct adequate barriers to prevent encroachment |

Table A.14	Parks and Protected Areas – Operation/Maintenance Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| - Routine use of highway - Maintenance of toll booths - Maintenance of structures | Increased unauthorised access to protected areas | Long Term | No interchanges that will allow unauthorised access to parks and protected areas. Ensure adequate boundaries to limit access. Enforcement of regulations |
| Increased access to parks & protected areas | Exceed carrying capacity of park | Long Term | Conduct carrying capacity studies. Know and enforce user limits. |
| Routine maintenance activity | Obstruction of proper park management strategies | Long Term | Proper storage of construction material within the park or protected area boundary. Removal of solid waste and sewage generated during the site preparation and construction phases, for disposal outside the protected area. Limit construction activities to within the right-of-way. Establishment and enforcement of Carrying Capacity for the park or protected area. Penalties for breaches in mitigation measures. |

Table A.15   Positive Impacts – Parks and Protected Areas

Potential Positive Impacts Some positive impacts are anticipated from the presence of the highway during the operation/maintenance phase. These include: Provision of adequate routes and access to parks and protected areas. Increase in public awareness of parks and protected areas through interpretive signs. Information gathering where required increasing the scientific database.

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Clearing during construction phase | Loss of primary forest and individual specimens | Short term activity with long term effect | Clear minimum required; make inventory of rare species; |
|  | Loss of secondary forest | Short term activity with long term effect | Identify areas where forestation with same species can take place; planting should be done before clearing if possible |
|  | Loss of forest resource and unknown species | Short term activity with long term effect | Conduct inventory and research before clearing |
|  | Loss of forest related economic activity, such as logging and harvesting of minor forest products | Short term | Harvest timber before clearing for construction. Reforest existing degraded lands to replace lost resource |
|  | Deforestation and related secondary impacts (watershed degradation) such as changes in: Groundwater, springs, wells Streamflow quantity Timing and distribution of streamflow On-site erosion Sediment in streams Nutrient outflow in stream water | Short term if slopes are reforested | During construction phase, put measures in place to reduce sediments yield to streams. Construct checks dams/gabions or other structures in gullies. Introduce reforestation as soon as possible after construction |
|  | Destabilization of slopes, causing or aggravating existing slope instability | Short term of slopes are reforested | Reforest slopes as soon as possible after construction. |
|  | Increased gully and stream bank erosion | Short term if slopes are reforested | Establish undisturbed riparian buffer zones |
| - Compaction by heavy machinery | Disturbance to soil; Compaction of soil; affects natural regeneration of forest | Long term | Tilling of soil to reverse compaction effects |

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Dust creation | Pollutant – stress to trees along corridor from excessive dust | Short term | Wetting of soil and trees to reduce effects of dust |
| Creation of open spaces in otherwise closed forest for actual corridor or for access roads | Fragmentation Decreases actual forest size Alienation of forest Reduces interaction with other communities Produces decline in number of species and abundance Loss of native species due to invasion/replacement by light or stress tolerant species Opening up of forest canopy can lead to changes in soil development. | Long term | Design criteria Minimise number of crossings through a forested area Avoid creating small non-viable portions of forest Reduce access to the forest from the main cut by installation of barriers |

Table A.17	Forestry Resources  - Operation/Maintenance Phase

| Activity | Negative Impact | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Fragmentation Creation of open spaces in otherwise closed forest for actual corridor or for access roads | Increases access; opening up of corridors to loggers, yam stick/charcoal traders and farmers | Long term | Creation of physical barriers along corridor to limit access Increase number of forest rangers/warders Create stiffer fines for encroachment |
|  | Remaining forest land may come under pressure for “development”, especially if economic value of land increases with presence of highway | Long term | Strict monitoring and enforcement of legislation. |
| Fragmentation Creation of open spaces in otherwise closed forest for actual corridor or for access roads | Fragment of forest more difficult to protect than large tract; yields easily to developmental pressure | Long term | Increase budgets allocated to wardens to ensure enforcement of existing legislation. Increase number of forest rangers/warders |
|  | The forest becomes more accessible to poachers. | Long term | Increase number of forest rangers/warders Create stiffer fines for encroachment |

| Activities | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Air pollution from vehicle exhaust | Gaseous and particulate pollutants from vehicular traffic. Gaseous emissions can be toxic to trees; particulates can smother leaves. | Long term | Monitor emissions from vehicles. Enforce restrictions on vehicles entering highway. Creation of buffer zone of vegetation between road and primary forest |
| Polluted runoff | Contamination of soil; stress on trees along corridor | Long term | Ditches and drains along highway to drain pollutants away from forest Creation of buffer zone between road and forest |
| Hazardous spills | Pollution of forest environment | Long term | Design ditches bordering highway to contain spills Rapid emergency response |
| - Relocation of residents displaced by highway | Relocated persons may make new clearings in forest for small farms | Long term | Strict enforcement of legislation and increases of fines. Increased monitoring of clearing/burning; |
| - Presence of Highway; all activities | Waldsterben or Waldschaden – Forest death or forest decline. The response of the forest to cumulative effect of many different stresses. | Long term |  |

Table A.18	Landscape Aesthetics and Scenic Vistas - Site Preparation/Construction Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Blasting & stripping Movement of heavy machinery Clearing forests, woodlands and cultivated areas. | Interruption of views | Long Term | Reduce Right-of Way to the minimum requirement. Landscape verges with vegetation compatible to existing flora. Views can be revealed, composed or reinforced by road layout and design. Speed must be taken into account. Construction of Lay-Bys with interpretive signs on upcoming views. Design criteria |
| Blasting & stripping Movement of heavy machinery Clearing forests, woodlands and cultivated areas. | Physical scarring of landscape |  | Minimise damage to aesthetic appeal by construction activity and design. Ensure proper altitudinal placement of road and associated structures. Re-forest and landscape scarred areas adjacent to the highway. Design criteria |

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Movement of heavy machinery Establishment of bridges and crossings | Interruption of physical and visual continuity | Long Term | Bridges, viaducts and tunnels can be used on steep terrain rather than high cut embankments to preserve continuity. Design criteria |
| Clearing of forests & woodland | Destruction of vegetation | Long Term | Reforestation of areas to replace lost acreage. Rehabilitation of adjacent areas. |
| Clearing of forests & woodland | Fragmentation of cultural areas | Long Term | Re-establishment of cultural zones. Design criteria |

Table A.19  Landscape Attributes and Scenic Vista -  Operation/Maintenance Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Establishment of right of way | Crossing or cutting off views of architectural or cultural heritage | Long Term | Ensure proper altitudinal placement of highway to facilitate views of sites. Indicate upcoming cultural/heritage views with interpretive signs. Construct Lay-Bys to allow for appreciation of views |
| Establishment of right of way | Visual scarring of landscape | Long Term | Prevent ‘uglification’ of the landscape. Harmonise design of structures with surrounding landscape. Regulate billboard use. Penalties and enforcement against littering. |
| Establishment of right of way | Interruption of topographical orientation | Long Term | Slopes can be varied to match the natural topography. |
| Lighting at night Illuminated billboards | Light Pollution | Long Term | Harmonise lighting at rest stops. Select and place lighting to reduce glare and negative visual impact. |
| Routine traffic Operation of toll booths Routine maintenance procedures | Littering | Long Term | Establish solid waste disposal systems. Penalties and enforcement of anti-litter laws. |

Table A.20  Positive Impacts -  Landscape Attributes and Scenic Vistas

#### Potential Positive Impacts Increased public access to and appreciation of vistas. Increased public awareness through interpretive signs and displays.

Table A.21  Natural Hazards – Site Preparation/Construction Phase

| Activity | Negative Impacts |  |  | Duration |  | Mitigation Measures |
| --- | --- | --- | --- | --- | --- | --- |
| Cutting | Undermining of slopes and resulting slope failure from cutting Spoil heaps could fail and become landslide hazards Changes in local groundwater levels could induce subsidence of sink holes and caverns |  |  | Long term |  | Slope stability measures put in place Spoil heaps carefully located, structured and monitored. |
| Blasting | Seismic effects – slope failure, collapse of caverns or sink holes |  |  | Short term |  | Dampening of blasting where possible |
| Filling | Overloading of slopes and resulting slope failure from filling Overloading of dolines and resulting collapse or subsidence. Can impact on many different types of land use. |  |  | Long term |  | Geotechnical investigation of slopes, slope stability measures Careful geophysical exploration and drilling to locate existing sinkholes and other potential subsidence sites. Cleaning and grouting of sinkholes and caverns. |
| - Overloading, undercutting and/or steepening of slopes |  | Changes in type, speed of failure, e.g. slow creep to progressive creep | Long term |  | Geophysical investigation and slope stability measures put in place |  |
| Construction of bridges |  | - Interference with permeability/mobility of banks – erosion and sedimentation during floods. | Long term |  | Careful hydrological investigations and bridge design to accommodate floods and mobility of banks. |  |

Table A.22	Natural Hazards – Operation/Maintenance Phase

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| - Impoundment of floodwaters upstream of highway | Inundation upstream | Long term | Adequate drainage under the road; ensure drainage pipes are large enough to accommodate flood flows. |
| Routing/channelisation /concentration of runoff | Flooding, gullying, debris flows in areas not normally flood prone. Drainage of runoff into sinks (whether intentional or not) can accelerate solution processes and induce subsidence | Long term | Careful drainage design Monitoring of drainage during operational phase Monitoring of sink holes |
| Floods and flash floods | Inundation, scouring of road surface, structural damage to road, bridges and other structures. | Long term | Design criteria Heightening and strengthening of levees Construction of artificial levees Canalization/straightening of streams Drainage diversion Channel regulation Bank revetment Dams across gullies Maintenance of flood control structures, clearing of drains. |

| Activity | Negative Impact | Duration | Mitigation |
| --- | --- | --- | --- |
|  | Slope failures and landslides can cover road and cause temporary effects only, Can damage surface and/or undermine entire road structure | Long term | Rapid response to clear debris and reopen road Adequate drainage to quickly remove water from slopes Partial slope removal, slope reduction, slope flattening or benching, depending on type of potential failure Construction of retaining walls, gabions. |
| #### Debris flows | Bridge undermining/collapse | Long term | Design criteria to allow debris flows to pass under road. |
| #### Rock Fall | Can be hazardous to drivers | Long term | Install rock traps at base of slope, Use mesh nets |

| Activity | Negative Impacts | Duration | Mitigation Measures |
| --- | --- | --- | --- |
| Natural subsidence due to solution or cavern collapse of existing dolines or sink holes induced by lowering of local ground water levels Cavern or sink hole collapse due to overloading of surface by fill. | Subsidence of road bed Tilting, cracking of surface Creation of collapse feature in road bed Structural failure of road | Long term | Geophysical exploration of karst terrain through which corridor runs. Compilation of sinkhole density maps Drilling to locate caverns Study of location and hydrology of all enclosed depressions (dolines, sinkholes) and caves. Avoid other land uses that may cause lowering of water table or other hydrological changes. Grouting /Infilling of caverns and sink holes. |
| Earthquake | Resulting small surface cracks to major structural damage. | Long term | Design criteria Maintenance of structures |
| Storm surge | Possible scour of bridge supports | Long term | Design criteria |

Table A.23   Positive Impacts – Natural Hazards

#### Potential Positive Impacts Reduced flood effects from drainage modification. Use of diverted run-off for irrigation. Installation of preventative slope-failure mechanisms.

Table A.24  Health and Safety – Site Preparation/Construction Phase

| Activity | Negative Impacts | Timeframe | Mitigation Measures |
| --- | --- | --- | --- |
| Cutting | Improper road geometry | Long Term | Refer to road design criteria |
| Cutting | Increase in levels of particulates | Short Term | Use masks and other respiratory equipment |
|  | Hazardous materials (HAZMAT) accidents | Medium Term | Establish and enforce safety procedures and also emergency response plans |
| Landscaping | Herbicides/pesticides | Long Term | Proper usage, education & storage |
| Drilling Blasting | Noise levels | Medium Term | Where activities occur outside of normal working hours, public relations and community consultations should be employed. In addition ear plugs and ear muffs should be used when noise exceeds 80 dB(A) for extended periods. |
| Transportation | Accidents | Medium Term | Use flagmen, safety signs, lights, reflectors and establish and enforce safety procedures example the wearing of hard hats and safety harness. |
| Blasting Drilling Cleaning | Digestive tract illnesses and reduction in hygiene standards |  | Adequate and clean storage facilities and water supply for cooking and washing, proper utensils. Ensure application of Food Handlers’ Permit. |
| Construction Companies | Improper disposal of sewage, solid waste and reduction in hygiene standards | Medium Term | Supply portable toilets and adequate water supply for showers and hand washing. |

Table A.24   Health and Safety – Operation/Maintenance Phase

| Activity | Negative Impacts | #### Duration | #### Mitigation Measures |
| --- | --- | --- | --- |
| Traffic use | Accidents caused by slippery or faulty road surface | Long Term | Use materials that reduce accidents, for example, asphalt that absorb water (porous). |
| Traffic use | Improper road geometry | Long Term | Design criteria. Avoid steep grades and sharp corners |
| Traffic use | Use of improper vehicles (old vehicles with harmful emissions, faulty vehicles, vehicles driving below the recommended speed limit) | Long Term | Legislation, education and enforcement |
| Traffic use | Increased traffic speed | Medium Term | Legislation, education and enforcement |
| Traffic use | Increased level of particulates | Long Term | Legislation, emissions testing and enforcement |
| Traffic use | Increase in levelsd of tail pipe and evaporative emissions | Long Term | Legislation, emissions testing and enforcement |
| Transportation | Hazardous materials (HAZMAT) accidents | Long Term | Legislation and emergency response plans. Establish and enforce safety procedures for transportation of HAZMAT. |
| Landscaping | Use of herbicides/pesticides in landscaping | Long Term | Proper storage and usage of materials and education and enforcement of existing legislation |
| Painting of Structures | Increase in levels of toxic materials for use of lead based paints for bridges | Long Term | Use lead free paint e.g. Zinc based paints |
| Traffic Use | Increased noise levels from motorized vehicles | Long Term | Legislation, noise barriers and enforcement where close to settlements |
| Traffic Use | Increase in accident | Long Term | Erection of adequate barriers to prevent unauthorized encroachment of the highway. Public education programme on proper use of the highway facilities (ramps, exits, entrances, speed limits, toll booths, etc. |
| Traffic Use | Increase levels of sanitary and solid waste | Long Term | Establish adequate sanitary and solid waste disposal facilities at convenient locations. These should be properly sited and of regulation standards. |

### Table A.25  Potential Negative and Positive Impacts- Economics Activities

| Negative Impacts | Mitigation Measures |
| --- | --- |
| #### Agriculture - Loss of productive lands, primarily sugar cane at Bushy Park, Innswood, and Worthy Park Estates; bananas on Clarendon Plains; coffee plantations in Manchester, citrus in St. James, and mixed farming in St. Ann, St. Elizabeth and St. James. - Destruction and modification of irrigation channels on the St. Catherine and Clarendon plains - Increased incidence of praedial larceny - Livestock wandering unto right-of-way - Opening up of hinterland to “opportunistic squatters” and landless farmers | Stakeholder consultations with estate managers and farmers who will be affected to determine scale of impact, and corrective measures to be taken. Adequate compensation packages will have to be worked out with the project-affected persons. Irrigation channels should be re-built, and an alternative source of water provided for these areas during the construction of these channels. Erection of structures to cross channels. Existing legislation on this practice should be revised and stiffer penalties applied to offenders. Erection of barriers and culverts for cattle crossing Restricted access by erection of barriers along corridor’s ROW, and enforcement of existing legislation |
| Potential Positive Impacts - Improved transport of goods resulting in greater access to internal and external markets #### - Potential development of market-gardening, reducing handling and storage of produce |  |

| Negative Impacts | Mitigation Measures |
| --- | --- |
| Small Business |  |
| - Loss of roadside business especially for stall vendors in Sandy Bay, Freetown, Clarendon Park and Whitney Turn -Encroachment of informal activities on highway ROW, especially in vicinity of Interchanges where traffic slows | Formalise and re-establish vendors in designated areas (lay-bys) to ensure continuity of business operations. These lay-bys should be constructed along the highway at strategic locations to accommodate those businesses (itinerant vendors) which depend solely on passers-by. Alternatively, the Faith’s Pen and Melrose Hill model could be adopted, and if this not feasible, alternative space should be provided within close proximity to te original vending sites. A detailed inventory and socio –economic survey should be carried out to determine the vendor population at risk. Appropriate landscaping and barriers to discourage encroachment. Development control and legislative measures should also be considered |
| Positive Impacts - Formal integration of small business operators into planned commercial activities, for example rest and food stops, craft and souvenir booths - Opportunity for food vendors standard to be established - Possible linkage between small business development and heritage tourism |  |
| Trade and Commerce |  |
| - Downturn in sales and patronage as towns are by-passed e.g. Old Harbour, Porus and Williamsfield | While the Interchanges will facilitate access to these towns, the level of business generated may not be guaranteed. However, if existing roads are upgraded and the level of services improved, these areas should continue to attract enough through traffic to maintain buoyancy in business operations. Local businesses should also be encouraged to take advantage of new business opportunities provided. |
| Positive Impacts - Generation of new businesses at Interchange points - Injection of private-sector skill/technology - Revitalization of dormant economies of some small towns and former growth centres such as Balaclava, Maggotty and Montpelier through increased access, greater visibility and increased traffic volumes |  |

| Negative Impacts | Mitigation Measures |
| --- | --- |
| Urban expansion |  |
| - Limitations to outward expansion and economic growth of towns such as Old Harbour as highway will occupy lands that could accommodate future growth of town | Buffer zone should be left between urban fence of towns and highway corridor to accommodate urban growth and expansion. |
| Positive Impacts The project can spur the planned development of towns to curtail uncontrolled urban sprawl, particularly into right-of-way of highway, and areas prone to disasters. |  |
| Mining |  |
| - Sterilisation of bauxite deposits in the Manchester area and Hope Flats and the Schwallenburgh/Gibralter area in St. Ann. -Existing mining and haulage roads may be altered or disrupted | Consultations with mining companies to ascertain future mining areas, and sites where high-grade ore exists. The sites that are likely to be impacted by the highway should be mined out prior to construction of the highway. Access to these private interior roads should be maintained by the erection of appropriate structures that will accommodate heavy duty trucks and trailers. |
| #### Tourism |  |
| - Opening up of interior areas to tourism could increase demand pressure on attractions such as YS Falls and Appleton Valley Tour, resulting in loss of amenity value and aesthetic appeal of these areas. - Uncontrolled growth of services and related industries can cause increased stress on community resources and infrastructure - Anti-tourism sentiments and visitor harassment | Carrying capacity studies should be conducted to determine acceptable limits change due to increased access and human interference. This is especially important for the potential opening up of pristine areas. These studies will inform the decision-making process with respect to linkages and connections, as well as appropriate legislation for regulating visitor use. Adequate infrastructure and services support to meet physical, social and economic needs of region should be provided to reduce stress on industry. The pace and scale of development of tourism-related activities should be controlled by appropriate integrated planning and legislation. Public awareness of industry sensitivity and benefits is required. The local community should also be involved in the decision-making process regarding tourism-development. |
| Potential Positive Impacts |  |
| Improved transportation network, and reduced commuting time for visitors moving along the east-west and north-south corridors, especially cruise ship passengers who are constrained by time. Enable access to wider range of attractions. Reduce pressure on existing attractions. Linkages with GOJ Strategic Master Plan for Sustainable Tourism Development and South Coast Development Plan through the development of cultural, heritage and community tourism Possible opening up of new areas in the interior of the island to tourism-related activities. Pristine areas in Manchester, St. Elizabeth, St. James and St. Ann offer potential sites for nature trails, picnics and other forms of recreational activities Potential stimulation of indigenous industries- for example art and craft work in areas like Maggotty and Balaclava, resulting in greater potential for income generation Support overall growth and development of industry. |  |

Table A.26 Community Structure – Site Preparation/construction and Operation/Maintenance Phases

| Negative Impacts | Mitigation Measures |
| --- | --- |
| Community Structure |  |
| Fragmentation of communities, resulting in their alienation from vital economic social services. These include Sandy Bay and Freetown in St. Catherine, Porus in Manchester and Gibralter, and Guanaboa Vale in St. Catherine. | The alignment should be repositioned to avoid fragmentation of communities. Where repositioning of the highway is not technically feasible, efforts should be made to re-connect such communities by the construction of pedestrian bridges, underpasses or overpasses. Vital services such as health facilities and schools should be centrally relocated within the community where easy accessibility is maintained. |
| #### Transportation |  |
| Access to livelihood - Alienation from life-line activities (economic space). Highway could alienate communities from earning a livelihood by cutting them off from income-generating activities | Minimum changes should be made to cut off access to life-line industries. Where existing roadways have to appropriated to facilitate the highway, alternative routes should be provided |
| #### Potential Positive Impacts |  |
| Reduction in commuting time, and lower vehicle-operating costs for motorists Overall infrastructural improvement in several communities across the island, especially those with an adequate road system An improved road system and installation of telecommunications conduits should also result in spinoffs for other sectors including health, education, commerce, industry and telecommunications |  |

| #### Culture shock |  |
| --- | --- |
| Superimposition of a highway on the rural landscape, and changes to community life could result in cultural shock, especially among the elderly | Community consultations will be necessary to sensitize citizens of the effects of building a highway through their community, to get their perception of the project and to garner support for its development. |
| Loss of roadside community/social activities Disruption to social and economic interaction which adds to community vitality | Provision of alternative venue for social activities, and facilitate the development of new town square. Community consultations to determine best options |
| #### Gentrification effect |  |
| Depressed land values due to pollution (noise, air), resulting in lack of high value investments of properties in close proximity to highway | Reduce air and noise pollution to minimum by application of appropriate air pollution mitigation strategies, and creation of sound barriers Maintain the aesthetic appeal of highway corridor and ROW |
| Potential Positive Impacts |  |
| Gentrification effect-property values in particular areas may increase due to infrastructural improvements resulting in higher lease and rental values. Improved road infrastructure is likely to induce new business ventures within the corridors, thus placing a premium on lands in close proximity to the highway. |  |

Table A.27  Land Acquisition and Settlement – Site Preparation/construction and Operation/Maintenance Phases

| Negative Impacts | Mitigation Measures |
| --- | --- |
| #### Loss of housing and lands |  |
| Individual houses will be expropriated in a number of communities in all three segments of the corridor resulting in forced displacement of persons | Resettlement and rehabilitation following detailed socio-economic inventory and survey to determine numbers of houses and properties that will be expropriated. Appropriate compensation to be applied according to GOJ standards. |
| Displacement of households |  |
| Involuntary displacement represents a break in social continuity, and may diminish people’s sense of control over their lives. It can also result in stress-related health problems, and the possible death of elderly folk unable to cope with the psychological trauma of resettling. | Resettlement sites are to meet the needs of the persons to be resettled. Host population and community should be consulted to ensure adequate preparation for receiving new entrants. Attempts should be made to settle persons in a social environment similar to the one they are leaving. Counselling will help to alleviate stress and assist in adjustment. |
| #### Land acquisition |  |
| This may result in the sub-division of family lands, and complex issues relating to land ownership and titles Severing of family ties to land | Adequate compensation package should be worked out for land acquisition based on value of properties. Affected persons without land titles should be assisted in acquiring them. |
| Temporary loss of income and separation from “economic space” Separation from lifeline activities which may induce impoverishment at the onset. | Comprehensive compensation for loss of income and economic support during the relocation exercise |
| Loss of customary rights to land and resources Squatters may face eviction although they have informal customary rights to the land they occupy (i.e. they have been living on the property undisturbed for at least 7 years) | Provision should be made for the resettlement of squatters, and the absence of titles to land should not be a barrier to compensation |
| #### Potential Positive Impacts |  |
| Improved living standards for many persons, especially those in impoverished areas who will benefit from higher standards of living and improved social services. |  |

### Table A.28  Culturally Sensitive Groups – Site Preparation/Construction and Operation/Maintenance Phases

| Negative Impacts | Mitigation Measures |
| --- | --- |
| Loss of traditional sense of identity. Breakdown of culturally distinct communities. Loss of livelihood. Loss of income. Increase in poverty and dependence on government. Breakdown of culturally distinct communities. Vanishing of traditional and distinct cultures. Cultural Disruption Loss of unique cultural aspects for Jamaica. | Communities as a whole in the project. Meet with representatives and take into consideration their concerns. Avoid creating links that are between culturally distinct communities and general population. Allow for sites along the highway where culturally distinct population can sell their craft and homegrown food. |

Table A.29   Cultural & Archaeological Heritage – Site Preparation/Construction Phase

| Negative Impacts | #### Mitigation Measures |
| --- | --- |
| Increased access to existing sites & new sites. Increased risk of damage to artefacts. Increased risk of modifying the integrity of the site. | Upon discovery of a new site, work should be stopped and the environmental supervisor should be contacted immediately. When a new site is discovered, access should be restricted until and archaeologist has been able to inspect the site and secure all valuable components. Design accessible display that limit risk of damage to artefacts. |
| Destruction of known sites Damages to components of the site. Loss of the context, when moving components of heritage site. Reduction of value of the heritage components. Destruction of heritage site and components Loss of unknown archaeological sites. Loss of clues to the understanding of past cultures and historical events. Loss of unique cultural insight for Jamaica’s past. | Any heritage site before specialists have evaluated it. Evaluate the possibility of moving the alignment to save the site. Avoid all unknown archaelogogical sites. Upon discovery of a new site, work should be stopped and the environmental supervisor should be contacted immediately. When a new site is discovered, access should be restricted until an archaeologist has been able to inspect the site and secure all valuable components. |

Table A.30   Positive Impacts - Cultural & Archaeological Heritage

#### Potential Positive Impacts Discovery of new archaeological sites Opportunities for interpretive sign-posting of sites Increased access to heritage tours